#************************************************************************

# check.def contains the tests, error thresholds, short warning messages

#   and text offering some explanation and advise issued by

#      the program PLATON when run in the 'VALIDATION' mode

#

#           This file is read when PLATON is called with the '-u' switch

#                    (e.g. 'platon -u sk1500.cif')

#

#   The file check.def should either be in the directory where the program

#   is run or specified with the environment variable CHECKDEF

#                  (e.g. setenv CHECKDEF /mnt/user/checkdef)

#

# - Validation output will be in the order as defined implicitly below

#

# - Tests are identified with three-digit numbers

#   _0xx - general

#   _1xx - cell/symmetry

#   _2xx - adp-related

#   _3xx - intra geometry

#   _4xx - inter geometry

#   _5xx - coordination geometry

#   _6xx - void tests

#   _7xx - varia

#   _8xx - (Fatal) Software Errors/Problems

#   _9xx - Reflection data issues

#

# - All tests are defined as two line stanzas

#   The first line gives the test-number and thresholds (A4, 3A5)

#      E.g. _klm crit_1 crit_2 crit_3

#   The second line defines the message (A55).

#      E.g. 'Structure contains solvent accessible Voids of $F A**3'

#

#   $I defines where (optionally) an 'integer' threshold/value will be placed

#   $F defines where (optionally) a 'real' threshold/value will be placed

#   $A defines where an atom label is inserted (first)

#   $B defines where an atom label is inserted (second)

#   $X defines where bond,angle,torsion (calculated) is inserted

#   $Y defines where bond,angle,torsion (reported)   is inserted

#

# Tests (value > crit) are done from right to left (ie crit_3 crit_2 crit_1)

#

# ALERT_Level_A = In General: Serious Problem.

# ALERT_Level_B = Potentially Serious Problem.

# ALERT_Level_C = Check & Explain.

# ALERT_Level_G = Issues to Check, Not Necessarily Errors.

#

# ALERT_Type_1  = CIF Construction/Syntax Error, Inconsistent or Missing Data.

# ALERT_Type_2  = Indicator that the Structure Model may be Wrong or Deficient.

# ALERT_Type_3  = Indicator that the Structure Quality may be Low.

# ALERT_Type_4  = Cosmetic improvement, Methodology, Query or Suggestion.

# ALERT_Type_5  = Informative Message, Check.

#

# NOTES: - Validation runs invoked with the '-u' option will provide textual

#          information and HELP on the reported ALERTS.

#        - Actual ALERTS start at _020

#***************************************************************************

# >>> Definition of Notification Levels (4A8)

_000                  A

ALERT_G ALERT_C ALERT_B ALERT_A

#---------------------------------------------------------------------------

# >>> Listing header on file *.chk (Pseudo Request)

_001                  A

# PLATON/CHECK-(------) versus check.def version of 120309

#---------------------------------------------------------------------------

# >>> Report Cell Data (Pseudo Request)

_002                  A

# CELL

#---------------------------------------------------------------------------

# >>> Report Space-Group  (Pseudo Request)

_003                  A

# Space-Group

#---------------------------------------------------------------------------

# >>> Report Moiety Formula (Pseudo Request)

_004                  A

# MoietyFormula

#---------------------------------------------------------------------------

# >>> Report Sum Formula (Pseudo Request)

_005                  A

# SumFormula

#---------------------------------------------------------------------------

# >>> Report MW

_006                  A

# MW

#---------------------------------------------------------------------------

# >>> Report Density

_007                  A

# Dx

#---------------------------------------------------------------------------

# >>> Report Z (Pseudo Request)

_008                  A

# Z

#---------------------------------------------------------------------------

#1>>> Report Mu (Pseudo Request)

_009                  A

# Mu

#---------------------------------------------------------------------------

#1>>> Report F000 (Pseudo Request)

_010                  A

# F000

#---------------------------------------------------------------------------

#1>>> Report Transmission (Pseudo Request)

_011                  A

# T

#---------------------------------------------------------------------------

#1>>> Report Estimated Transmission (Pseudo Request)

_012                  A

# T

#---------------------------------------------------------------------------

#1>>> Report Hmax,Kmax,Lmax, Theta(max), N(obs), (Pseudo Request)

_013                  A

# THM

#---------------------------------------------------------------------------

#1>>> Report Hmax,Kmax,Lmax, N(calc), Ratio (Pseudo Request)

_014                  A

# THM

#---------------------------------------------------------------------------

#1>>> Report Refinement results

_015                  A

# R

#---------------------------------------------------------------------------

#1>>> Check Rint

_020 0.12 0.18 0.25 3 A

The value of Rint is greater than 0.12 ......... $F

The value of Rint (i.e. _diffrn_reflns_av_R_equivalents) should normally be

considerably less than 0.12 and in the order of magnitude of the reported

R-values. Rint may be relatively meaningless when based on a very limited

number of averaged data. Higher values should be accompanied by a suitable

explanation in the _publ_section_exptl_refinement section. However,

authors should first ensure that there are not overlooked problems

associated with the data or the space-group. Elevated values for

_diffrn_reflns_av_R_equivalents may be indicative of a need to recollect

the data from a crystal of higher quality or that there is a problem with

the data treatment. Consider the following:

(a) The absorption corrections are inadequate or inappropriate.

(b) The overall quality of the data may be poor due to the crystal quality.

(c) The crystal is very weakly diffracting, so that a large proportion of

essentially "unobserved" reflections are being used in the refinement.

You should consider using a better crystal or a data collection at

low temperature and/or, if the compound is organic, using Cu radiation.

(d) You are working in the wrong crystal system or Laue group.

(e) You have only a very small number of equivalent reflections, which

may lead to artificially high values of _diffrn_reflns_av_R_equivalents

Note that if _diffrn_reflns_av_sigmaI/netI is also large, the quality

of the data should be considered to be suspect.

#---------------------------------------------------------------------------

# >>> Check Expected number of Reflections (Max = 1 Centro, 2 - non-centro)

_021 0.01 0.05 1.00 1 A

Ratio Unique / Expected Reflections too High ... $F

The expected number of reflections corresponds to that in the asymmmetric

unit of the Laue group. Expected ratio: less-or-equal 1 for centro symmetric

structures and less than 2 for non-centrosymmetric structures.

Reasons to exceed those numbers can be:

1 - Systematic extinctions not omitted from the obsd data count.

2 - Refinement with redundant (i.e. not merged/unique) data set.

#---------------------------------------------------------------------------

# >>> Check Expected number of Reflections

_022 0.05 0.10 0.15 3 A

Ratio Unique / Expected Reflections too Low .... $F

Test for data completeness. The ratio of the reported number of unique

reflections and expected number of reflections for the resolution given is

reported. The ratio can be low due to a missing cusp of data when collected

with a 2D-detector. Alternatively, the wrong asymmetric part of reciprocal

space was collected on a serial detector system.

#---------------------------------------------------------------------------

# >>> Check Theta-Max

_023 .010 .025 0.05 3 A

Resolution (too) Low [sin(theta)/Lambda < 0.6].. $F Deg.

Check resolution of the data set. Alert is issued when sin(theta)/lambda < 0.6

#---------------------------------------------------------------------------

# >>> Check for required Friedel pair averaging Z<=Si

_024    0    2    2 4 I

Merging of Friedel Pairs is Indicated ..........          !

Averaging of Friedel pairs is strongly advised when indicated by a

large su on the Flack parameter. Large su values indicate that the

anomalous scattering power is too small in combination with the data

quality at hand to merit refinement with a non-averaged dataset. This

will generally be the case with MoKa datasets for structures containing

atoms not heavier than Si. The value of the Flack parameter will be

largely meaningless anyway for large su values.

Use MERG 4 in case of refinement with the SHELXL97 program.

Non-compliance with the above for valid scientific reasons should be

discussed in detail in the experimental section of the paper.

#---------------------------------------------------------------------------

# >>> Check for Hmin..Lmax

_025 .000 .000 0.00 1 A

Hmin..Lmax Data Incomplete or Missing ..........          ?

Check reported h,k,l - range with calculated range based on reported theta-max.

#---------------------------------------------------------------------------

# >>> Check for weak data

_026   50   60   70 3 A

Ratio Observed / Unique Reflections too Low .... $I Perc.

Check whether a sufficient fraction of the unique data is indeed above the

2 sigma level.

#---------------------------------------------------------------------------

# >>> Check _diffrn_reflns_theta_full

_027 .005 .005 .006 3 A

_diffrn_reflns_theta_full (too) Low ............ $F Deg.

Ideally (and a requirement for publication in Acta Crystallographica),

the dataset should be essentially complete, as defined by

-diffrn-measured-fraction-theta-full (close to 1.0), up to

sin(theta)/lambda = 0.6 (i.e. 25.24 degrees MoKa).

The three major causes of incomplete data sets are:

1 - A missing cusp of data due to datacollection by rotation around

the spindle axis only (standard on some image-plate systems).

Cure: collect an additional data set after remounting the crystal.

2 - The DENZO imageprocessing package has problems with certain strong

reflections. They are often excluded from the data set.

Cure: Add an additional scan at lower powersetting in order to

include strong low order reflections.

3 - Incomplete scans.

#---------------------------------------------------------------------------

# >>> Check _diffrn_measured_fraction_theta_max

_028 .050 .100 0.15 3 A

_diffrn_measured_fraction_theta_max  Low ....... $F

Ideally, the reported '_diffrn_measured_fraction_theta_max' value,

corresponding to theta-max, should be close to 1.0.

#---------------------------------------------------------------------------

# >>> Check _diffrn_measured_fraction_theta_full

_029 .020 .040 .060 3 A

_diffrn_measured_fraction_theta_full Low ....... $F

Ideally (and a requirement for publication in Acta Crystallographica),

this fraction should be close to 1.0 for theta-full greater or equal

to sin(theta/lambda) = 0.6 (i.e. 25.24 degrees for MoKa and 67.7 degrees

for CuKa  radiation).

The three major causes of incomplete data sets are:

1 - A missing cusp of data due to datacollection by rotation around

the spindle axis only (standard on some image-plate systems).

Cure: collect an additional data set after remounting the crystal.

2 - The DENZO imageprocessing package has problems with certain strong

reflections. They are often excluded from the data set.

Cure: Add an additional scan at lower powersetting in order to

include strong low order reflections.

3 - Incomplete scans, possibly based on erroneously assumed higher than

actual symmetry.

Note: The default value of _diffrn_measured_fraction_theta_full that

is automatically calculated and inserted in the CIF by SHELXL-97 might

generate A-level ALERTS when significant numbers of reflections are

missing at higher theta values. In order to avoid such an ALERT,

substitute the values calculated with the SHELXL instruction 'ACTA 50'

for _diffrn_reflns_theta_full and _diffrn_measured_fraction_theta_full

respectively. For Mo-radiation, corresponding values of 25 degees (or

higher) and 0.99 (or higher) are expected.

PLATON may be used to analyse the case at hand (by invoking either the

'FCF-VALIDATION' mode or the 'ASYM-VIEW' mode).

#---------------------------------------------------------------------------

# >>> Check _diffrn_reflns_number > reflns_number_total

_030    0    2    2 1 A

_diffrn_reflns_number .LE.  _reflns_number_total          ?

The number of measured reflections should be equal or greater than the

number of unique reflections.

#---------------------------------------------------------------------------

# >>> Check need for Extinction Correction Parameter

_031  0.3  0.4  1.0 4 A

Refined Extinction Parameter within Range ...... $F Sigma

This test checks whether a refined extinction parameter is meaningful

i.e. whether its value is significantly larger than its corresponding s.u.

If not, this parameter should be removed from the model and the structure

refined without this meaningless additional parameter.

The current default gives a warning when its value is within 3.33 s.u.

SHELXL97-2 will not allow negative values leading to ill-convergence and

non-zero maximum shift/error values: remove extinction parameter from the

refinement.

#---------------------------------------------------------------------------

# >>> Check su Flack Parameter

_032  0.2 10.0 10.0 4 A

Std. Uncertainty in Flack Parameter too High ... $F

Check the validity of the absolute structure determination.

A high su indicates that the experimental data do not support the

determination of the absolute structure. This will generally be the case

with light atom MoKa data where f" is nearly zero.

Note: Use the TWIN & BASF 0.0 instructions in SHELXL97. The default

FLACK parameter is not always reliable, in particular when strongly

correlated with the position of the origin (e.g. along y in space-group P21).

Please refer to Flack,H.D. & Bernardinelli, G. (1999) Acta Cryst. A55,

908-915 and (2000) J. Appl. Cryst., 33, 1143-1148.

#---------------------------------------------------------------------------

# >>> Check Flack Parameter value

_033  0.3 10.0 10.0 4 A

Flack x Parameter Value Deviates from Zero ..... $F

Check the relevance/validity of the absolute structure determination.

Please refer to Flack,H.D. & Bernardinelli, G. (1999) Acta Cryst. A55,

908-915 and (2000) J. Appl. Cryst., 33, 1143-1148.

A value of the Flack parameter that deviates significantly from zero

(taking into account the associated s.u.) might indicate that the

absolute structure should be inverted in case of a value closer to 1.0

than to zero. A value close to 0.5 may be indicative of an inversion

twin or a missed centre of inversion.

For valid absolute structure assigments, abs(x) should be less thans 2 * su,

with su < 0.04. For enantiopure compounds, su should be less than 0.1.

#---------------------------------------------------------------------------

# >>> Check for Flack parameter value specified Z>Si, non-centro

_034    0    2    2 1 A

No Flack Parameter Given. Z .GT. Si, NonCentro .          ?

No Flack parameter value given for non-centrosymmetric structure with

heaviest atom Z > Si. This might be intentional.

#---------------------------------------------------------------------------

# >>> Check for _chemical_absolute_configuration

_035    0    0    0 1 A

No _chemical_absolute_configuration info given .          ?

Options are 'rm', 'ad', 'rmad', 'syn', 'unk' or '.'

rm   : absolute configuration established by the structure determination

of a compound containing a chiral reference molecule of known

absolute configuration.

ad   : absolute configuration established by anomalous dispersion effects

in diffraction measurements on the crystal.

rmad : absolute configuration established by the structure determination

of a compound containing a chiral reference molecule of known

absolute configuration and confirmed by anomalous dispersion

effects in diffraction measurements on the crystal.

syn  : absolute configuration has not been established by anomalous

dispersion effects in diffraction measurements on the crystal.

The enantiomer has been assigned by reference to an unchanging

chiral centre in the synthetic procedure.

unk  : absolute configuration is unknown, there being no firm chemical

evidence for its assignment to hand and it having not been

established by anomalous dispersion effects in diffraction

measurements on the crystal. An arbitrary choice of enantiomer

has been made.

.    : inapplicable.

#---------------------------------------------------------------------------

# >>> Check for missing Flack Parameter su

_036  0.5  0.5  0.5 1 A

No s.u. Given for Flack Parameter ..............          ?

No standard uncertainty found for the Flack parameter.

When the structure refinement was done with SHELXL97-2, the likely

reason for this is a missing BASF instruction. This applies in

particular when the associated Flack parameter has the value 0.000.

No valid conclusions on the absolute structure can be drawn.

#---------------------------------------------------------------------------

# >>> Check _diffrn_reflns_theta_full

_037    0    0    0 1 I

_diffrn_reflns_theta_full .......... Not Given .          ?

No information is given about the theta value for which the dataset is

complete, subject to the percentage given with the dataname

_diffrn_measured_fraction_theta_full.

#---------------------------------------------------------------------------

# >>> Check _diffrn_measured_fraction_theta_max

_038    0    0    0 1 I

_diffrn_measured_fraction_theta_max  Not Given .          ?

This fraction should be specified in combination with the theta value

given with the dataname _diffrn_reflns_theta_full.

#---------------------------------------------------------------------------

# >>> Check _diffrn_measured_fraction_theta_full

_039    0    0    0 1 I

_diffrn_measured_fraction_theta_full Not Given .          ?

This fraction should be specified in combination with the value for theta-max.

#---------------------------------------------------------------------------

# >>> Test for H-atoms [0,1]

_040  0.0 99.0 99.0 1 A

No H-atoms in this Carbon Containing Compound ..          ?

Alert for 'no H-atoms' in CIF. This is unusual for carbon containing compounds,

but may be correct.

#---------------------------------------------------------------------------

# >>> Test SumFormula

_041  0.0 99.0 99.0 1 A

Calc. and Reported SumFormula    Strings  Differ          ?

In the ideal case, both SumFormula strings (reported and calculated) should

be identical. If not, the reason for the difference should be clear. Examples

are cases where populations do not add up to integer numbers, or when solvent

molecules have been SQUEEZED.

Note: SHELXL97 reports population parameters in the CIF with two decimals

only. This may lead to non-integer atom counts in cases of disorder

due to rounding.

Note: Alerts _041, _042 & _045 can probably be ignored when the relevant

values differ by the same factor.

#---------------------------------------------------------------------------

# >>> Test MoietyFormula

_042  0.0 99.0 99.0 1 I

Calc. and Reported MoietyFormula Strings  Differ          ?

In the ideal case, the MoietyFormula string as reported should be identical

to the MoietyFormula string calculated from the data in the CIF. If not, the

reason should be clear. Examples are cases where there is no separating space

between two element names or cases where populations do not add up to

integer numbers or when moieties are separated by '.' instead of ','.

Example: NO3 should be given as N O3

Note: Alerts _041, _042 & _045 can probably be ignored when the relevant

values differ by the same factor.

#---------------------------------------------------------------------------

#1>>> Test for MolWeight

_043  1.0  5.0 10.0 1 A

Check Reported Molecular Weight ................ $F

Note: atomic weights used in the calculation of the molecular weight are

taken from Inorg. Chim. Acta 217 (1994) 217-218 which deviate in a few cases

slightly from the older values used in SHELXL97-2.

#---------------------------------------------------------------------------

#1>>> Check Reported with calculated density

_044  1.0  5.0 10.0 1 A

Calculated and Reported Dx Differ ..............          ?

In the ideal case, both data items should be the same within a small

tolerance. If not, the reason should be clear.

#---------------------------------------------------------------------------

#1>>> Check Reported and Calculated Z

_045  0.0  1.0  1.0 1 A

Calculated and Reported Z Differ by ............ $F Ratio

In the ideal case, both data items (Z(calc) & Z(reported)) should be

the same. If not, the reason for the difference should be clear. An example

is the situation where PLATON gives Z = 1 when the program cannot work out

a proper Z.

Note: Alerts _041, _042 & _045 can probably be ignored when the relevant

values differ by the same factor.

#---------------------------------------------------------------------------

#1>>> Check Reported Density with calculated density from Z*MW

_046  1.0  5.0 10.0 1 A

Reported Z, MW and D(calc) are Inconsistent .... $F

D(calc) as calculated from the reported Z and MW is compared for consistency

with the reported d(calc).

#---------------------------------------------------------------------------

#1>>> Test SumFormula Given

_047  0.0  0.0  0.0 1 A

SumFormula Not Given ...........................          ?

The Sumformula, corresponding with the Moietyformula, should be given.

#---------------------------------------------------------------------------

#1>>> Test MoietyFormula Given

_048  0.0 99.0 99.0 1 I

MoietyFormula Not Given ........................          ?

The Moiety formula (i.e. the specification of the various species in the

structure) should be given in the CIF.

Example: '(Cd 2+)3, (C6 N6Cr 3-)2, 2(H2 O)'

#---------------------------------------------------------------------------

# >>> Check Calculated Density .GT. 1.0

_049  0.0  0.0  1.0 1 A

Calculated Density less than 1.0 gcm-3 ......... $F

The calculated density will with a few exceptions be larger than 1.0.

A smaller value may indicate either an incomplete model or incorrect

symmetry. (e.g. a missing 'bar' in P-1 etc.)

#---------------------------------------------------------------------------

# >>> Test for mu given [0,1]

_050  0.0  0.0  0.0 1 A

Absorption Coefficient mu Missing ..............          ?

The linear absorption coefficient corresponding to the Sumformula should be

given.

#---------------------------------------------------------------------------

# >>> Test for difference mu(cif) with mu(calc) [%]

_051    1    5   10 1 A

Mu(calc) and Mu(CIF) Ratio Differs from 1.0 by . $F Perc.

In the ideal case, both data items should be the same within a small

tolerance. If not, the reason should be clear.

#---------------------------------------------------------------------------

# >>> Test for specification absorption correction method [0,1]

_052  0.0  0.0  0.0 1 A

(Proper) Absorption Correction Method Missing ..          ?

The treatment/method of absorption(correction) should be given explicitly.

#---------------------------------------------------------------------------

# >>> Test for specification xtal_dimension_min [0,1]

_053  0.0  0.0  0.0 1 I

Minimum Crystal Dimension Missing (or Error) ...          ?

The smallest crystal dimension should be supplied in the CIF.

The expected value should be a real number (i.e. not 0.35mm)

#---------------------------------------------------------------------------

# >>> Test for specification xtal_dimension_mid [0,1]

_054  0.0  0.0  0.0 1 I

Medium  Crystal Dimension Missing (or Error) ...          ?

The medium crystal dimension should be supplied in the CIF.

The expected value should be a real number (i.e. not 0.35mm)

#---------------------------------------------------------------------------

# >>> Test for specification xtal_dimension_max [0,1]

_055  0.0  0.0  0.0 1 I

Maximum Crystal Dimension Missing (or Error) ...          ?

The largest crystal dimension should be supplied in the CIF.

The expected value should be a real number (i.e. not 0.35mm)

#---------------------------------------------------------------------------

# >>> Test for specification xtal_radius [0,1]

_056  0.0  0.0  0.0 1 I

Crystal Radius Missing .........................          ?

#---------------------------------------------------------------------------

# >>> Test for correction for absorption needed

_057  1.1  1.2  1.3 3 A

Correction for Absorption Required   RT(exp) ... $F

Correction for absorption is likely to be needed.

#---------------------------------------------------------------------------

# >>> Test for specification Tmax [0,1]

_058  0.5  0.5  0.5 1 A

Maximum Transmission Factor Missing ............          ?

The Maximum transmission factor should be specified in case a correction

for absorption was done. This is NOT the value that is calculated

automatically with SHELXL when a SIZE instruction is given in the SHELXL

instruction file. The values reported by SHELXL represent the EXPECTED

correction range. Some correction packages (e.g. SADABS) will provide

only one 'relativer- correction-factor'. In such cases, Tmax should be

given as Tmax-expected and Tmin = relative-correction-factor * Tmax.

#---------------------------------------------------------------------------

# >>> Test for specification Tmin [0,1]

_059  0.5  0.5  0.5 1 A

Minimum Transmission Factor Missing ............          ?

The Minimum transmission factor should be specified in case a correction

for absorption was done. This is NOT the value that is calculated

automatically with SHELXL when a SIZE instruction is given in the SHELXL

instruction file. The values reported by SHELXL represent the EXPECTED

correction range. Some correction packages (e.g. SADABS) will provide

only one 'relative-correction-factor'. In such cases, Tmax should be

given as Tmax-expected and Tmin = relative-correction-factor * Tmax.

#---------------------------------------------------------------------------

# >>> RR Test

_060 1.10 1.50 2.00 4 I

Ratio Tmax/Tmin (Exp-to-Rep) (too) Large ....... $F

see IUCR WEB-Pages

#---------------------------------------------------------------------------

# >>> RR' Test

_061 0.10 0.25 0.50 4 I

Tmax/Tmin Range Test RR' too Large ............. $F

see IUCR WEB-Pages

#---------------------------------------------------------------------------

# >>> Rescale Tmin & Tmax

_062    0    2    2 4 I

Rescale T(min) & T(max) by ..................... $F

Some (empirical) correction packages (e.g. SADABS) will provide only one

'relative-correction-factor'. In such cases, Tmax should be given as

Tmax-expected (as calculated from the crystal dimensions) and

Tmin = relative-correction-factor * Tmax.

#---------------------------------------------------------------------------

# >>> Test for Crystal Size

_063  0.6  0.8  1.0 4 I

Crystal Probably too Large for Beam Size ....... $F mm

Alert for crystals with at least one dimension probably too large for the

homogeneous part of the X-ray beam when used for datacollection using

crystal monochromated radiation. An exception will be datacollection using

a beta-filter and a sufficiently large collimator.

#---------------------------------------------------------------------------

# >>> Test for T(max) .GE. T(min)

_064  0.0  0.0  0.0 1 A

Reported T(min) is Greater than Reported T(max) .         ?

The order of Tmin and Tmax values are likely to be interchanged.

#---------------------------------------------------------------------------

# >>> Test for applicability of (semi-)empirical abs.corr. [0,1]

_065  3.0  3.0  3.0 3 A

Crystal Requires Numerical Correction mu*tmid .. $F

For high mu * r values, numerical absorption correction procedures are

recommended.

#---------------------------------------------------------------------------

# >>> Test whether Predicted and Reported Transmission Ranges are Identical

_066  0.0  2.0  2.0 1 I

Predicted and Reported Transmissions Identical .          ?

The predicted and reported transmission ranges are found to be identical

which is not to be expected. CIF's generated with SHELXL97 report

transmission ranges based on the crystal dimensions supplied on the

SIZE card. Those values have nothing to do with the actual corrections

for absorption as applied to the data: they just report the EXPECTED range.

Some correction packages (e.g. SADABS) will provide only one

'relative-correction-factor'. In such cases, Tmax should be given as

Tmax-expected and Tmin = relative-correction-factor * Tmax.

#---------------------------------------------------------------------------

# >>> Insure that minimum dimension less max dimension

_067  0.0  0.0  0.0 1 I

Maximum Dimension Less Min. Xtal Dimension .....          ?

Minimum an Maximum dimensions are likely exchanged in the CIF.

#---------------------------------------------------------------------------

# >>> Test for F000 Calc/Reported difference

_068  0.0 10.0 10.0 1 I

Reported F000 Differs from Calcd (or Missing)...          ?

In the ideal case, both data items should have the same value. If not,

the reason should be clear. A reason might be the output by SHELXL-97

of population parameters to the CIF with only two decimals.

Note: SHELXL counts the number of electrons in the unit cell. The result

will in general be an integer. This is also the number checked for here.

The official definition calls for 'The effective number of electrons in the

crystal unit cell contributing to F(000)'. It may contain dispersion

contributions and is calculated as:

F(000) = [ (sum f~r~)^2^ + (sum f~i~)^2^ ]^1/2^

f~r~ = real part of the scattering factors at theta = 0

f~i~ = imaginary part of the scattering factors at theta = 0

#---------------------------------------------------------------------------

# >>> Test for label without numerical part

_069  0.0  2.0  2.0 1 I

Atom Label without Numerical Part ..............    $A

Acta Cryst. Notes for Authors requires atom labels to contain a numerical

part. E.g. A label of the type 'O' should be given as 'O1'.

This is not necessarily a requirement for other journals.

#---------------------------------------------------------------------------

# >>> Test for duplicate labels

_070  0.0  0.0  0.0 1 A

Duplicate Atomic Label on INPUT ................    $A

The CIF contains duplicate labels posing interpretation problems for

PLATON/CHECK. Derived geometry ALERTS may have their origin in this

problem.

#---------------------------------------------------------------------------

# >>> Test for uninterpretable labels

_071    0    0    0 1 A

Uninterpretable Atom Label on Input ............    $A

The CIF contains labels posing problems for PLATON/CHECK.

Example: label HN1 with no scattering type information supplied.

Validation is aborted.

#---------------------------------------------------------------------------

# >>> Test for inconsistency 'constr' versus 'H-Atoms refined'

_073    0    0    0 1 I

H-atoms ref, but _hydrogen_treatment reported as    $A

The structure contains refined hydrogen atoms. However the data item

_refine_ls_hydrogen_treatment has the value 'constr'.

The value 'mixed' is more appropriate.

#---------------------------------------------------------------------------

# >>> Test for Occupancy equal 0.0

_074  0.0  1.0  1.0 1 A

Occupancy Parameter = 0.0 for ..................    $A

The CIF contains an atom with occupacy less than 0.0001

#---------------------------------------------------------------------------

# >>> Test for Occupancy greater than 1.0

_075  0.0  0.0  0.0 1 A

Occupancy $F greater than 1.0 for ......    $A

The CIF contains an atom with Occupancy greater than 1.0.

#---------------------------------------------------------------------------

# >>> Test for Occupancy less than 1.0 for atom on special position

_076  0.0  1.0  1.0 1 A

Occupancy $F less than 1.0 for Sp.pos  .    $A

The CIF contains an atom sitting on a special position with occupancy

specified as less than 1.0. This is often an error and the result of the

confusion of the notions 'occupancy' and 'population parameter'. The first

should be 1.0 for a fully occupied site. The latter multiplies the

site-symmetry with the occupancy. Thus, for a fully occupied site on a

mirror plane the site-symmetry will be 0.5 * 1.0 = 0.5.

Note: a wrong occupancy number will lead to an incorrect expected

chemical formula.

#---------------------------------------------------------------------------

# >>> Test for Non-Integral # of atoms in Unit Cell

_077  0.0  1.0  1.0 4 A

Unitcell contains non-integer number of atoms ..          ?

The unitcell contains a non-integer number of atoms of a given atom type.

Valid reasons include partially occupied (solvent) sites and substitutional

disorder.

#---------------------------------------------------------------------------

# >>> Test for inconsistency 'geom' versus 'no H-Atoms'

_078    0    0    0 1 I

No H-atoms, but _solution_hydrogens reported as     $A

The structure contains no hydrogen atoms. However the data item

_atom_sites_solution_hydrogen had the value 'geom'. This value is

likely the SHELXL default and should be replaced by '.'.

#---------------------------------------------------------------------------

# >>> Test for inconsistency 'mixed' versus 'no H-Atoms'

_079    0    0    0 1 I

No H-atoms, but _hydrogen_treatment reported as     $A

The structure contains no hydrogen atoms. However the data item

_refine_ls_hydrogen_treatment has the value 'mixed'. This value is

likely the SHELXL default and should be replaced by '.'.

#---------------------------------------------------------------------------

# >>> Test maximum shift/error

_080 0.05 0.10 0.20 2 A

Maximum Shift/Error ............................ $F

Convergence of the refinement is proven with a close to zero shift/error

value for all refined parameters. Such a convergence is easily achieved

with a few additional refinement cycles at little cost.

Note: Some SHELXL-97 versions do not allow for negative Flack parameter

values. Convergence in such a case may be never reached because the

Flack parameter value is reset to zero.

#---------------------------------------------------------------------------

# >>> Test for  maximum shift/error given

_081 0.00 0.00 0.00 1 A

No Maximum Shift/Error Given ...................          ?

A maximum shift/error should be specified in order to judge convergence.

#---------------------------------------------------------------------------

# >>> Test for reasonable R1

_082 0.10 0.15 0.20 2 A

High R1 Value .................................. $F

A higher than usual R1 indicates either an insufficient model or poor

quality data.

#---------------------------------------------------------------------------

# >>> Test for extreme second weighting parameter (SHELXL)

_083  5.0 25.0 50.0 2 A

SHELXL Second Parameter in WGHT unusually Large. $F

The second parameter on the SHELXL weighting line has an exceptionally

large value. This may indicate either improper reflection s.u.'s or

an unresolved problem such as missed twinning.

#---------------------------------------------------------------------------

# >>> Test for reasonable wR2

_084 0.25 0.35 0.45 2 A

High R2 Value .................................. $F

R2 will in general have a value twice of that of R1 with refinement on F**2.

Significantly larger values usually indicate a poor refinement model.

Also check for unaccounted for twinning.

#---------------------------------------------------------------------------

# >>> Test for default SHELXL weighting scheme

_085    0   10   10 2 A

SHELXL default weighting scheme is not optimized          ?

The weighting scheme is found to be left at the SHELXL default.

#---------------------------------------------------------------------------

# >>> Test for reasonable S  (Too Low)

_086  0.4  0.6  0.8 2 A

Unsatisfactory S Value (Too Low or Not Given) .. $F

S should in general be close to 1 at the end of a refinement with a proper

weighting scheme. If not, there might be significant unresolved problems

with the model.

#---------------------------------------------------------------------------

# >>> Test for reasonable S (Too High)

_087  2.0  4.0  6.0 2 A

Unsatisfactory S value (Too High) .............. $F

S should in general be close to 1 at the end of a refinement with a proper

weighting scheme. If not, there might be significant unresolved problems

with the model.

#---------------------------------------------------------------------------

# >>> Test for reasonable Data / parameter ratio  (centro)

_088 10.0 12.5 16.7 3 A

Poor Data / Parameter Ratio .................... $F

The data/parameter ratio should in general be higher than 10 for a quality

structure determination. This ratio can be improved by not refining

C-H parameters other than riding on their carrier atom.

#---------------------------------------------------------------------------

# >>> Test for reasonable Data / parameter ratio (non-centro) (Zmax < 18)

_089 12.5 16.7 25.0 3 A

Poor Data / Parameter Ratio (Zmax .LT. 18) ..... $F

The data/parameter ratio should in general be higher than 8 for a quality

determination of a structure containing atoms with Z less than 18.

This ratio can be improved by not refining C-H parameters other than riding

on their carrier atom.

Note that with light atom non-centrosymmetric structures where anomalous

dispersion effects are insignificant, it is unwise to attempt to use

unmerged Friedel-related reflections simply to boost the r/p ratio.

#---------------------------------------------------------------------------

# >>> Test for reasonable Data / parameter ratio (non-centro) (ZMAX > 18)

_090 10.0 12.5 16.7 3 A

Poor Data / Parameter Ratio (Zmax .GT. 18) ..... $F

The data/parameter ratio should in general be higher than 10 for a quality

determination for a structure containing heavy atoms with ZMAX greater

than 17. This ratio can be improved by not refining C-H parameters other

than riding on their carrier atom.

#---------------------------------------------------------------------------

# >>> Test for 'No-wavelength given'

_091    0    0    0 1 A

No Wavelength found in CIF - 0.71079 Ang Assumed          ?

No Wavelength specification found in the CIF.

#---------------------------------------------------------------------------

# >>> Test for wavelength type [Cu,Mo,Ag]

_092    0    0    0 4 A

Check: Wavelength given is not Cu,Mo or Ag Ka .. $F Ang.

Warning: specified wavelength is not Cu,Mo or Ag Ka radiation.

Valid exceptions are Neutron and Synchrotron data.

#---------------------------------------------------------------------------

# >>> Test for inconsistency 'mixed' versus 'no refined H'

_093    0    0    0 1 I

No su's on H-atoms, but refinement reported as .    $A

The 'mixed' type Hydrogen atom refinement is reported (SHELXL-97 default).

However, no Hydrogen atoms with freely refined positions are found in the

CIF. Likely, the value 'constr' for '_refine_ls_hydrogen_treatment' will

be more appropriate (e.g. when all Hydrogen atoms have been refined in

the riding mode on their carrier atom).

#---------------------------------------------------------------------------

# >>> Test for maximum/minimum residual density ratio

_094  2.0  4.0  8.0 2 A

Ratio of Maximum / Minimum Residual Density .... $F

The ratio of the maximum and minimum residual density excursions is

unusual. This might indicate unaccounted for twinning or missing atoms

(e.g. associated with disordered solvent).

#---------------------------------------------------------------------------

# >>> Test for residual density maximum given [0,1]

_095    0    0    0 1 A

No Residual Density Maximum Given ..............          ?

No residual electron density maximum given in CIF.

#---------------------------------------------------------------------------

# >>> Test for residual density minimum given [0,1]

_096    0    0    0 1 A

No Residual Density Minimum Given ..............          ?

No residual electron density minimum given in CIF.

#---------------------------------------------------------------------------

# >>> Test maximum residual density

_097 0.75  1.0  2.0 2 A

Large Reported Max. (Positive) Residual Density. $F eA-3

Residual density maximum larger than expected. This might be caused by

residual absorption artifacts, unaccounted for twinning, wrongly assigned

atom types and other model errors.

#---------------------------------------------------------------------------

# >>> Test for minimum residual density

_098 0.75  1.0  2.0 2 A

Large Reported Min. (Negative) Residual Density. $F eA-3

Residual density minimum larger than expected. This might be caused by

residual absorption artifacts, wrongly assigned atom types and other

model errors.

#---------------------------------------------------------------------------

# >>> Test for minimum residual density less zero [0, 1]

_099  0.0  0.0  0.0 1 A

Minimum (Negative) Residual Density .GE. 0 !!... $F eA-3

Likely interchanged maximum and minimum values.

Alternatively, the minimum resididual density has the (unlikely) value

zero.

#===========================================================================

# >>> Test for insufficient digits for special pos 1/3 & 2/3 in x-coord.

_101    0    0   10 2 A

Limited Precision x-coordinate  $A Suspect:      $B

Fractions of the type 1/3 and 2/3 for the positional parameters of

atoms in special positions should be provided with sufficient digits.

(i.e. 0.66667 and 0.33333).

#===========================================================================

# >>> Test for insufficient digits for special pos 1/3 & 2/3 in x-coord.

_102    0    0   10 2 A

Limited Precision y-coordinate  $A Suspect:      $B

Fractions of the type 1/3 and 2/3 for the positional parameters of

atoms in special positions should be provided with sufficient digits.

(i.e. 0.66667 and 0.33333).

#===========================================================================

# >>> Test for insufficient digits for special pos 1/3 & 2/3 in x-coord.

_103    0    0   10 2 A

Limited Precision z-coordinate  $A Suspect:      $B

Fractions of the type 1/3 and 2/3 for the positional parameters of

atoms in special positions should be provided with sufficient digits.

(i.e. 0.66667 and 0.33333).

#===========================================================================

# >>> Test for additional translational symmetry [0, 1]

_110  0.5  0.5  999 2 A

ADDSYM Detects Potential Lattice Centering or Halving .   ?

Tests for missed symmetry are done with ADDSYM, an expanded MISSYM (C) clone.

These tests warn for missed or possible higher (pseudo) symmetry in the

structural model (i.e. based on the coordinate data).

Close examination of the situation at hand is indicated in order to

prove/disprove the issue (usually in combination with the reflection data).

Report on potential (pseudo/real) lattice centering or cell halving.

Note: H-atoms and disordered atoms are not taken into account in the tests.

#---------------------------------------------------------------------------

# >>> Test for additional centre of symmetry [0, 100]

_111   50   50  100 2 A

ADDSYM Detects (Pseudo) Centre of Symmetry ..... $I PerFit

Tests for missed symmetry are done with ADDSYM, an expanded MISSYM (C) clone.

These tests warn for missed or possible higher (pseudo) symmetry in the

structural model (i.e. based on the coordinate data). Close examination of

the situation at hand is indicated in order to prove/disprove the issue

(usually in combination with the reflection data).

This ALERT reports on a potential additional (pseudo/real) inversion centre.

A pseudo-centre may be incompatibel with existing symmetry elements.

Chiral molecules are incompatible with an inversion centre.

Note: H-atoms and disordered atoms are not taken into account in the test.

#---------------------------------------------------------------------------

# >>> Test for additional symmetry [0, 1]

_112   50   50  100 2 A

ADDSYM Detects Additional (Pseudo) Symm. Elem...    $A

Tests for missed symmetry are done with ADDSYM, an expanded MISSYM (C) clone.

These tests warn for missed or possible higher (pseudo) symmetry in the

structural model (i.e. based on the coordinate data). Close examination of

the situation at hand is indicated in order to prove/disprove the issue

(usually in combination with the reflection data).

This ALERT reports on potential additional (pseudo/real) rotation axes and

mirrors.

In addition, (pseudo/real) lattice centering/translations are reported as

A, B, C, I, X, Y, Z, S. (Here S stands for special and not covered by the

preceeding types). Full details on the situation at hand should be gleaned

from an actual PLATON/ADDSYM run.

Chiral molecules are incompatible with an inversion centre or (glide)planes.

Note: H-atoms and disordered atoms are not taken into account in the tests.

#===========================================================================

# >>> Report New space-group suggested by ADDSYM

_113   90   95   99 2 A

ADDSYM Suggests Possible Pseudo/New Space-group.    $A

Tests for missed symmetry are done with ADDSYM, an extended MISSYM (C) clone.

These tests warn for missed or possible higher (pseudo) symmetry in the

structural model (i.e. based on the coordinate data). Close examination of

the situation at hand is indicated in order to prove/disprove the issue

(usually in combination with the reflection data).

Chiral molecules are incompatible with an inversion centre or (glide)planes.

For an example of reported pseudo-symmetry see I.A.Guzei et al, (2002).

Acta Cryst. C58, m141-m143.

Note: H-atoms and disordered atoms (i.e. atoms with population less

than 1.0) are not taken into account in the tests. This may artificially

lead to a symmetry higher than the actual one.

Note: Atoms are treated as having the same atom type in order to catch

certain types of disorder or incorrect atom type assignment.

#===========================================================================

# >>> Report on ADDSYM problem

_114    0    2    2 2 A

ADDSYM Could not (Re)Construct Proper Spacegroup          !

ADDSYM has problems to reconstruct a space-group from the symmetry

operation found in the symmetry expanded coordinate set. The reason being

either intricate additionaly detected pseudo-symmetry or serious errors

in the data set.

#---------------------------------------------------------------------------

# >>> Test for non-crystallographic centre of symmetry [0, 100]

_115   50   50  100 5 A

ADDSYM Detects Noncrystallographic Inversion ... $I PerFit

Tests for missed symmetry are done with ADDSYM, an expanded MISSYM (C) clone.

This ALERT reports on local inversion symmetry, not compatible with the

reported space-group symmetry.

Note: H-atoms and disordered atoms are not taken into account in the test.

#---------------------------------------------------------------------------

# >>> Report Problem with symmetry operator syntax

_119    0    0    0 1 A

Problem with the Syntax of a Symmetry Operation. $A

A symmetry operation should be specified in the CIF either without

spaces or between quotes.

#===========================================================================

# >>> Test for consistent _symmetry_space_group_name_H-M and Symm Opp

_120    0   10   10 1 A

Reported SPGR $A Inconsistent with Explicit      $B

Space-group symmetry should be provided in the CIF both explicitly with a

_symmetry_equiv_pos_as_xyz loop and implicitly with

_symmetry_space_group_name-H-M.

An unusual (non-standard) choice of origin may also raise this ALERT.

Please check and Explain.

#===========================================================================

# >>> Test for valid _symmetry_space_group_name_H-M

_121    0   10   10 1 A

Invalid _symmetry_space_group_name_H-M .........       $A

Symmetry in the CIF should be provided both explicitly with a

_symmetry_equiv_pos_as_xyz loop and implicitly with

_symmetry_space_group_name_H-M.

Test for valid _symmetry_space_group_name_H-M symbol.

#===========================================================================

# >>> Test for ? _symmetry_space_group_name_H-M

_122    0    0    0 1 A

No _symmetry_space_group_name_H-M Given ........          ?

Symmetry in the CIF should be provided both explicitly with a

_symmetry_equiv_pos_as_xyz loop and implicitly with

_symmetry_space_group_name_H-M.

Test for missing (i.e. ?) _symmetry_space_group_name_H-M symbol.

#===========================================================================

# >>> Test for Interpretable Space-Group Symmetry

_123    0    0    0 1 A

Uninterpretable or Inconsistent Space-Group Info.         ?

Symmetry in the CIF should be provided in the CIF both explicitly with a

_symmetry_equiv_pos_as_xyz loop and implicitly with

_symmetry_space_group_name_H-M.

Test for uninterpretable or inconsistent Space-group information.

#===========================================================================

# >>> Test for _symmetry_equiv_pos_as_xyz present

_124    0    0    0 1 A

Uninterpretable or Absent _symmetry_equiv_pos_as_xyz .    ?

Symmetry in the CIF should be provided in the CIF both explicitly with a

_symmetry_equiv_pos_as_xyz loop and implicitly with

_symmetry_space_group_name_H-M.

Test for uninterpretable or absent explicit symmetry records

#===========================================================================

# >>> Test for ? _symmetry_space_group_name_Hall

_125    0   10   10 4 I

No _symmetry_space_group_name_Hall Given .......          ?

Optionally specify the Hall symbol. The Hall symbol provides an

unambiguous definition of the space-group symmetry where the Hermann-

Mauguin symbol leaves room for alternative choises of the origin.

E.g. for space-group P21, the screw axis is in general taken to coincide

with the b-axis. However, sometimes it is choosen to be shifted by 1/4

in the c-axis direction to bring out the relation with P21/c. The Hall

symbols will be 'P 2yb' and 'P 2ybc' respectively.

Refer to: S.R.Hall, Space-Group Notation with an Explicit Origin;

Acta Cryst. (1981), A37, 517-525.

or: http://www.kristall.ethz.ch/LFK/software/sginfo/hall_symbols.html

#===========================================================================

# >>> Test for  _symmetry_space_group_name_Hall error

_126    0   10   10 1 A

Error in or Uninterpretable Hall Symbol  .......    $A$B

The reported Hall-symbol is found to be in error or uninterpretable.

Refer to: S.R.Hall, Space-Group Notation with an Explicit Origin;

Acta Cryst. (1981), A37, 517-525.

or: http://www.kristall.ethz.ch/LFK/software/sginfo/hall_symbols.html

#===========================================================================

# >>> Test for  _symmetry_space_group_name_Hall consistency

_127    0   10   10 1 A

Implicit Hall Symbol  Inconsistent with Explicit    $A$B

The reported Hall-symbol is found to be inconsistent with the one derived

from the explicit symmetry operations.

Alternatively, no Hall-symbol could be derived by PLATON for the explicit

set of symmetry operations. This may be the case when an unusual origin

is choosen.

Refer to: S.R.Hall, Space-Group Notation with an Explicit Origin;

Acta Cryst. (1981), A37, 517-525.

or: http://www.kristall.ethz.ch/LFK/software/sginfo/hall_symbols.html

#===========================================================================

# >>> Test for non-standard monoclinic space-group setting

_128    0   10   10 4 A

Non-standard setting of Space-group $A ....      $B

The reported monoclinic space-group is in a non-standard setting.

Transformation to the conventional setting is indicated unless there

is a good (scientific) reason not to do so.

#===========================================================================

# >>> Test for unusual non-standard Space-group name

_129    0    0    0 4 A

Unusual Space-group Specified ..................    $A

The reported space-group name is unusual.

#===========================================================================

# >>> Test for Cubic: a = b = c

_130    0    0    0 1 A

Cubic : a, b & c Dimensions Differ ...........            ?

Symmetry constraints on cell dimensions are checked.

#===========================================================================

# >>> Test for Cubic: alpha = beta = gamma = 90

_131    0    0    0 1 A

Cubic : alpha, beta and gamma should be 90 Deg Exact      ?

Symmetry constraints on cell dimensions are checked.

#===========================================================================

# >>> Test for Trigonal/Hexagonal : a = b

_132    0    0    0 1 A

Trigonal/Hexagonal a and b Differ ..............          ?

Symmetry constraints on cell dimensions are checked.

#===========================================================================

# >>> Test for Trigonal/Hexagonal : alpha = beta = 90

_133    0    0    0 1 A

Trigonal/Hexagonal alpha and beta should be 90 Deg Exact  ?

Symmetry constraints on cell dimensions are checked.

#===========================================================================

# >>> Test for Trigonal/Hexagonal : gamma = 120

_134    0    0    0 1 A

Trigonal/Hexagonal gamma should be 120 Deg Exact          ?

Symmetry constraints on cell dimensions are checked.

#===========================================================================

# >>> Test for Tetragonal: a = b

_135    0    0    0 1 A

Tetragonal: a and b should be Equal ...........           ?

Symmetry constraints on cell dimensions are checked.

#===========================================================================

# >>> Test for Tetragonal: alpha = beta = gamma = 90

_136    0    0    0 1 A

Tetragonal: alpha, beta & gamma should be 90 Deg Exact    ?

Symmetry constraints on cell dimensions are checked.

#===========================================================================

# >>> Test for Orthorhombic: alpha = beta = gamma = 90

_137    0    0    0 1 A

Orthorhombic: alpha, beta & gamma should be 90 Deg Exact  ?

Symmetry constraints on cell dimensions are checked.

#===========================================================================

# >>> Test for Monoclinic more than 1 angle off 90 degrees

_138    0    0    0 1 A

Monoclinic: More than one Angle Unequal 90.0 ...          ?

Symmetry constraints on cell dimensions are checked.

#===========================================================================

# >>> Test for Rhombohedral a = b = c

_139    0    0    0 1 A

Rhombohedral: a, b & c are not all Equal Exactly          ?

Symmetry constraints on cell dimensions are checked.

#===========================================================================

# >>> Test for Rhombohedral alpha = beta = gamma

_140    0    0    0 1 A

Rhombohedral: alpha, beta & gamma are Not All Equal       ?

Symmetry constraints on cell dimensions are checked.

#===========================================================================

#1>>> su on a - axis small or missing

_141    0   10   10 4 A

su on a - Axis Small or Missing (x 100000) ..... $I Ang.

The su on the a-axis is small or missing. The presence of su's (where

required) and value are checked. Su's as given by the diffractometer

software are often much smaller then realistic.

#===========================================================================

#1>>> su on b - axis small or missing

_142    0   10   10 4 A

su on b - Axis Small or Missing (x 100000) ..... $I Ang.

The su on the b-axis is small or missing. The presence of su's (where

required) and value are checked. Su's as given by the diffractometer

software are often much smaller then realistic.

#===========================================================================

#1>>> su on c - axis small or missing

_143    0   10   10 4 A

su on c - Axis Small or Missing (x 100000) ..... $I Ang.

The su on the c-axis is small or missing. The presence of su's (where

required) and value are checked. Su's as given by the diffractometer

software are often much smaller then realistic.

#===========================================================================

#1>>> su on alpha small or missing

_144    0   10   10 4 A

su on alpha    Small or Missing  (x 10000) ..... $I Deg.

The su on alpha is small or missing. The presence of su's (where required)

and value are checked. Su's as given by the diffractometer software are

often much smaller then realistic.

#===========================================================================

#1>>> su on beta  small or missing

_145    0   10   10 4 A

su on beta     Small or Missing  (x 10000) ..... $I Deg.

The su on beta is small or missing. The presence of su's (where required)

and value are checked. Su's as given by the diffractometer software are

often much smaller then realistic.

#===========================================================================

#1>>> su on gamma small or missing

_146    0   10   10 4 A

su on gamma    Small or Missing  (x 10000) ..... $I Deg.

The su on gamma is small or missing. The presence of su's (where required)

and value are checked. Su's as given by the diffractometer software are

often much smaller then realistic.

#===========================================================================

#1>>> su on symmetry restricted cell angle

_147    0   10   10 1 A

su on Symmetry Constrained Cell Angle(s) .......          ?

There should be no s.u. on symmetry constrained cell angles.

Example: No su on alpha, beta and gamma for orthorhombic symmetry.

#===========================================================================

#1>>> su on a,b, or c - axis too large

_148    0    0   10 3 A

su on the $A - Axis is Too Large (x 1000) . $I Ang.

The su on the reported -axis is unexpectedly large.

#===========================================================================

#1>>> su on alpha, beta or gamma too large

_149    0    0   10 3 A

su on the $A  Angle is Too Large (x 100) .. $I Deg.

The su on the reported angle is too large.

#===========================================================================

#1>>> Check Volume

_150 0.5   1.0  1.0 1 A

Volume as Calculated Differs from that Given ... $F Ang-3

An ALERT is issued when the reported unit cell volume differs significantly

from the volume calculated on the basis of the supplied cell dimensions.

#===========================================================================

# >>> Check for s.u. on Volume

_151    0   10   10 1 A

No su (esd) Given on Volume ....................          ?

Missing s.u. on cell volume.

#===========================================================================

# >>> Check for consistency of su on Volume and cell parameters

_152    0   10   10 1 I

Supplied and Calc Volume s.u. Inconsistent .....          ?

Some software packages calculate Volume su's incorrectly. The correct formula

(based on the propagation of error expression) may be found in:

M. Nardelli, Computer & Chemistry, (1983), 7, 95-98.

or

C. Giacovazzo ed. in 'Fundamentals of Crystallography', Second Edition,

Oxford University Press, 2003,  p135.

#===========================================================================

# >>> test for equal axial su's

_153    0   10   10 1 I

The su's on the Cell Axes   are Equal (x 100000) $I Ang.

The reported cell axes su's are reported equal. Please check whether

this is correct or a software default value.

#===========================================================================

# >>> test for equal cell angle su's

_154    0   10   10 1 I

The su's on the Cell Angles are Equal  (x 10000) $I Deg.

The reported cell angle su's are reported equal. Please check whether

this is correct or a software default value.

#===========================================================================

# >>> Check for reduced cell aP

_155    0   10   10 4 A

The Triclinic Unitcell is NOT Reduced ..........          ?

Unless for special reasons related to the structure/content, a unitcell and

structure is best reported with reference to the Niggli Reduced Cell.

This ALERT may originate also from a failure to order the axes from

small to large.

#===========================================================================

# >>> Check for non-standard axial order

_156    0   10   10 4 A

Axial System Input Cell not Standard ...........          ?

The axial order should be from small to large in the triclinic cell.

#===========================================================================

# >>> Check for non-standard monoclinic beta angle less 90 Degrees.

_157  0.0  10.  10. 4 A

Non-standard Monoclinic Beta Angle less 90 Deg   $F Deg.

By convention, the Monoclinic beta angle is always chosen to be larger than

90.0 Degrees. A trivial transformation (1 0 0/0 -1 0/0 0 -1) should be

applied to the data.

#===========================================================================

# >>> Check for standard reduced cell

_158    0   10   10 4 A

The Input Unitcell is NOT Standard/Reduced .....          ?

Unless for special reasons related to the structure/content, a unitcell and

structure is best reported with reference to the Niggli Reduced Cell.

#===========================================================================

# >>> Missing x-coordinate su

_161    0   10   10 4 A

Missing or Zero su (esd) on x-coordinate for ...    $A

Missing or Zero su (esd) on x-coordinate.

Positional parameters for all non-hydrogen atoms in general positions are

checked for the presence of a non-zero s.u. on them. This includes

parameters fixed to fix the origin in polar space-groups which is no longer

necessary when refinement is done with modern programs (e.g. SHELXL, XTAL).

#===========================================================================

# >>> Missing y-coordinate su

_162    0   10   10 4 A

Missing or Zero su (esd) on y-coordinate for ...    $A

Missing or Zero su (esd) on y-coordinate.

Positional parameters for all non-hydrogen atoms in general positions are

checked for the presence of a non-zero s.u. on them. This includes

parameters fixed to fix the origin in polar space-groups (e.g. P21) which

is no longer necessary when refinement is done with modern programs

(e.g. SHELXL, XTAL).

#===========================================================================

# >>> Missing z-coordinate su

_163    0   10   10 4 A

Missing or Zero su (esd) on z-coordinate for ...    $A

Missing or Zero su (esd) on z-coordinate.

Positional parameters for all non-hydrogen atoms in general positions are

checked for the presence of a non-zero s.u. on them. This includes

parameters fixed to fix the origin in polar space-groups (e.g. P41) which

is no longer necessary when refinement is done with modern programs

(e.g. SHELXL, XTAL).

#===========================================================================

# >>> Check for Refined C-H H-Atoms

_164    0   10   10 4 A

Nr. of Refined C-H H-Atoms in Heavy-Atom Struct. $I

Warning: Refined C-H H-atoms in heavy-atom structure (i.e. containing an

element beyond element #18). Such H-atoms are in general better refined

at calculated positions riding on the atoms they are attached to. A

better data over parameter ratio will be achieved.

#===========================================================================

# >>> Check for R-flagged Non-H Atoms

_165    0   10   10 3 A

Nr. of Status R Flagged Non-Hydrogen Atoms ..... $I

Report on restrained (riding) Non-Hydrogen atoms. Note: This may lead to

non meaningfull bond and angle su's (ALERTS _751, _752).

R-flagged atoms may arise unintensional being caused by an "AFIX 0" line

being missing in a shelxl.ins file (SHELXL-97 refinement).

Alternatively, the number of refined parameters may have been limited

deliberately (e.g. by refinement of C-F with fixed known geometry,

similar to C-H) in order to keep the data/parameter ratio acceptable.

#===========================================================================

# >>> Check for calc flagged atoms with s.u.s on coordinates

_166    0   10   10 4 A

S.U's Given on Coordinates for calc-flagged ....    $A

Calc-flagged atoms are not supposed to carry s.u.'s on their coordinates.

#===========================================================================

# >>> Check for sufficient data in in Atom data loop

_170    0    0    0 4 A

Insufficient Data in Coordinate loop ...........    $A

Insufficient data encountered in coordinate loop.

A possible cause might be the use of a SHELX style '=' continuation line.

#===========================================================================

# >>> Check Rounding of Cell Axes and Angles

_180    0    0    0 4 I

Check Cell Rounding: # of Values Ending with 0 = $I

It is unusual that more cell parameters end with a zero and the su is 10.

This problem might be caused by the specification of insufficient

'meaningful' digits as compared to the reported su.

see also: W.Clegg, Acta Cryst. (2003) E59, e2-e5.

#===========================================================================

# >>> Check for missing _cell_measurement_reflns_used datum

_194    0   10   10 1 I

Missing _cell_measurement_reflns_used datum ....          ?

Please supply value for _cell_measurement_reflns_used

#===========================================================================

# >>> Check for missing _cell_measurement_theta_max datum

_195    0   10   10 1 I

Missing _cell_measurement_theta_max   datum ....          ?

Please supply value for _cell_measurement_theta_max

#===========================================================================

# >>> Check for missing _cell_measurement_theta_min datum

_196    0   10   10 1 I

Missing _cell_measurement_theta_min   datum ....          ?

Please supply value for _cell_measurement_theta_min

#===========================================================================

# >>> Test for specification of unitcell measurement temperature

_197    0    0    0 1 I

Missing _cell_measurement_temperature ..........          ?

Please specify the temperature (Kelvin) at which the unit cell was

determined.

#===========================================================================

# >>> Test for specification of Datacollection temperature

_198    0    0    0 1 A

Missing _diffrn_ambient_temperature ............          ?

Please specify the temperature (Kelvin) at which the intensity data were

collected.

#===========================================================================

# >>> Test for SHELXL Roomtemperature Default   (Cell)

_199    0    0    0 1 I

Check the Reported _cell_measurement_temperature $I K

The cell determination temperature is set in the CIF by default by SHELXL to

293 K if the TEMP instruction is not used. The actual temperature is likely

either slightly or significantly (for a low temperature data collection)

different.

#===========================================================================

# >>> Test for SHELXL Roomtemperature Default (Datacollection)

_200    0    0    0 1 I

Check the Reported _diffrn_ambient_temperature . $I K

The data collection temperature is set in the CIF by default by SHELXL to

293 K if the TEMP instruction is not used. The actual temperature is likely

either slightly or significantly (for a low temperature data collection)

different.

#===========================================================================

# >>> Test for isotropic non-H atoms in main residue(s)

_201    0    0   10 2 A

Isotropic non-H Atoms in Main Residue(s) ....... $I

This test reports on non-hydrogen atoms that were refined with isotropic

displacement parameters only in the main residue. Such a practice is unusual

by modern standards and only needed for minor disorder modelling.

#---------------------------------------------------------------------------

# >>> Test for isotropic non-H atoms in anion ? or solvent ?

_202    0   50   50 3a A

Isotropic non-H Atoms in Anion/Solvent ......... $I

This test reports on isotropically refined atoms in small moieties (usually

anions or solvent).

#---------------------------------------------------------------------------

# >>> Test for 'all-isotropic adp(s)

_210    0    0   10 3 A

No Anisotropic ADP's Found in CIF ..............          ?

No anisotropically refined atoms in CIF ?

#---------------------------------------------------------------------------

# >>> Test for NPD ADP's  (1.0) in main residue(s)

_211    0    0  0.5 2 A

ADP of Atom $A is NPD .....................          ?

This test reports on non-positive definite (i.e. complex and unrealistic)

anisotropic displacement parameters in the main residue.

#---------------------------------------------------------------------------

# >>> Test for NPD ADP's in anion? & solvent ? [0, 1]

_212    0  0.5   99 2 A

ADP of Atom $A is NPD .....................          ?

This test reports on non-positive definite (i.e. complex and

unrealistic) anisotropic displacement parameters in an anion or solvent

residue.

#---------------------------------------------------------------------------

# >>> Test ratio adp max/min in main residue(s)

_213  3.0  4.0  5.0 2 A

Atom $A has ADP max/min Ratio ............. $F $B

The maximum and minimum main axis ADP ratio (Angstrom Units) is tested for

the main residue. Large values may indicate unresolved disorder.

#---------------------------------------------------------------------------

# >>> Test ratio adp max/min in anion ? or solvent ?

_214  4.0  5.0  6.0 2 A

Atom $A (Anion/Solvent) ADP max/min Ratio   $F $B

The maximum and minimum main axis ADP ratio (Angstrom Units) is tested for

the minor residue(s). Large values may indicate unresolved disorder.

#---------------------------------------------------------------------------

# >>> Test for unusual disordered atom ADP in main residue

_215  3.0  4.0  5.0 3 I

Disordered $A has ADP max/min Ratio ....... $F

The maximum and minimum main axis ADP ratio (Angstrom Units) is tested for

the main residue. Large values may indicate unresolved disorder.

#-------------------------------------------------------------------------

# >>> Test for unusual disordered atom ADP in minor residue

_216  5.0  7.0  9.0 3 I

Disordered $A (An/Solv) ADP max/min Ratio   $F

The maximum and minimum main axis ADP ratio (Angstrom Units) is tested for

the minor residue(s). Large values may indicate unresolved disorder.

#---------------------------------------------------------------------------

# >>> Test for Incomplete UIJ data

_217  0.0  0.0  0.0 1 A

Incomplete U(aniso) data for ...................    $A

Check & Correct U(aniso) data for completeness etc.

Do not use SHELX style '=' continuation line

#---------------------------------------------------------------------------

# >>> Test Ueq(max)/Ueq(Min) range for non-H atoms in non-solvent

_220  2.5  3.5  4.5 2 I

Large Non-Solvent   $AUeq(max)/Ueq(min) ... $F Ratio

This test reports on a larger than usual U(eq) range for the specified

element type in the non-solvent/anion part of the structure.

Too high or too low Ueq's may be an indication for incorrectly

identified atomic species (i.e. O versus N).

#---------------------------------------------------------------------------

# >>> Test Ueq(max)/Ueq(Min) range for non-H atoms in solvent

_221  3.0  4.0  5.0 4 A

Large Solvent/Anion $AUeq(max)/Ueq(min) ... $F Ratio

This test reports on a larger than usual U(eq) range for the non-hydrogen

atoms solvent/anion. Too high or too low Ueq's may be an indication for

incorrectly identified atomic species (i.e. Br versus Ag).

#---------------------------------------------------------------------------

# >>> Test Ueq(max)/Ueq(Min) range for H atoms in non-solvent

_222  3.0  4.0  5.0 3 A

Large Non-Solvent    H     Ueq(max)/Ueq(min) ... $F Ratio

This test reports on a larger than usual range of U(eq) values for hydrogen

atoms in the non-solvent/anion part of the structure.

Possibe causes are:

1 - disorder, e.g. in t-butyl moieties.

2 - poor data, not adequate for the refinement of individual displacement

parameters.

3 - Misplaced hydrogen atoms (i.e. there is no density at the position

where one of the H-atoms is positioned).

#---------------------------------------------------------------------------

# >>> Test Ueq(max)/Ueq(Min) range for H atoms in solvent

_223  3.0  4.0  5.0 4 A

Large Solvent/Anion  H     Ueq(max)/Ueq(min) ... $F Ratio

This test reports on large ranges in displacement parameters for hydrogen

atoms in the solvent/anion part of the structure.

#---------------------------------------------------------------------------

# >>> Test for difference in implicit and explicit U(eq)

_224 .002 .003 .005 1 A

Ueq(Rep) and Ueq(Calc) differ by$A Ang**2 .        $B

This test reports on a large difference between Ueq in the CIF and the Ueq

calculated from the 6 reported Uij values.                            .

#---------------------------------------------------------------------------

# >>> Hirshfeld Rigid-Bond Test (Acta Cryst (1976),A32,239-244

_230  5.0  7.0 99.0 2 A

Hirshfeld Test Diff for    $A--  $B .. $F su

The components of the anisotropic displacement parameters along chemical

bonds are assumed to be equal in magnitude. Large differences supposedly

indicate contamination of these parameters with other (unresolved) effects

such as (substitutional) disorder, model or data errors and/or

over refinement. Atomic sites assigned the wrong scattering type (e.g. Ag

versus Br) should generate 'problem signals' with this test. Data sets

corrected for absorption effects with DELREF techniques (e.g. DIFABS, SHELXA,

XABS2) often show large DELU values for bonds involving the heaviest atom.

Note: The original 'Hirshfeld-test' was defined in absolute terms (see

F.L.Hirshfeld, Acta Cryst. (1976). A32, 239-244). The current test is with

reference to the associated standard uncertainty.

#---------------------------------------------------------------------------

# >>> Hirshfeld Rigid-Bond Test (Acta Cryst (1976),A32,239-244

_231  5.0  9.0 99.0 4 A

Hirshfeld Test (Solvent)   $A--  $B .. $F su

The components of the anisotropic displacement parameters along chemical

bonds are assumed to be equal in magnitude. Large differences supposedly

indicate contamination of these parameters with other (unresolved) effects

such as (substitutional) disorder, model or data errors and/or

over refinement. Atomic sites assigned the wrong scattering type (e.g. Ag

versus Br) should generate 'problem signals' with this test. Data sets

corrected for absorption effects with DELREF techniques (e.g. DIFABS, SHELXA,

XABS2) often show large DELU values for bonds involving the heaviest atom.

Note: The original 'Hirshfeld-test' was defined in absolute terms (see

F.L.Hirshfeld, Acta Cryst. (1976). A32, 239-244). The current test is with

reference to the associated standard uncertainty.

#---------------------------------------------------------------------------

# >>> Hirshfeld Rigid-Bond Test (Metal-X) (Acta Cryst (1976),A32,239-244

_232  5.0 10.0 99.0 2 A

Hirshfeld Test Diff (M-X)  $A--  $B .. $F su

The components of the anisotropic displacement parameters along chemical

bonds are assumed to be equal in magnitude. Large differences supposedly

indicate contamination of these parameters with other (unresolved) effects

such as (substitutional) disorder, model or data errors and/or

over-refinement. Atomic sites assigned the wrong scattering type (e.g. Ag

versus Br) should generate 'problem signals' with this test. Data sets

corrected for absorption effects with DELREF techniques (e.g. DIFABS, SHELXA,

XABS2) often show large DELU values for bonds involving the heaviest atom.

A special case are M-C=O type of systems that generally show significant

differences for the M-C bond. See D.Braga & T.F. Koetzle (1988), Acta Cryst.

B44, 151-155).

Note: The original 'Hirshfeld-test' was defined in absolute terms (see

F.L.Hirshfeld, Acta Cryst. (1976). A32, 239-244). The current test is with

reference to the associated standard uncertainty.

#---------------------------------------------------------------------------

# >>> Hirshfeld Rigid-Bond Test (Metal-X) (Acta Cryst (1976),A32,239-244

_233 10.0 15.0 99.0 4 A

Hirshfeld (M-X Solvent)    $A--  $B .. $F su

The components of the anisotropic displacement parameters along chemical

bonds are assumed to be equal in magnitude. Large differences supposedly

indicate contamination of these parameters with other (unresolved) effects

such as (substitutional) disorder, model or data errors and/or

over refinement. Atomic sites assigned the wrong scattering type (e.g. Ag

versus Br) should generate 'problem signals' with this test. Data sets

corrected for absorption effects with DELREF techniques (e.g. DIFABS, SHELXA,

XABS2) often show large DELU values for bonds involving the heaviest atom.

Note: The original 'Hirshfeld-test' was defined in absolute terms (see

F.L.Hirshfeld, Acta Cryst. (1976). A32, 239-244). The current test is with

reference to the associated standard uncertainty.

#---------------------------------------------------------------------------

# >>> Hirshfeld Rigid-Bond Test (Acta Cryst (1976),A32,239-244

_234 0.15 0.25  0.3 4 A

Large Hirshfeld Difference $A--  $B .. $F Ang.

The components of the anisotropic displacement parameters along chemical

bonds are assumed to be equal in magnitude. Large differences supposedly

indicate contamination of these parameters with other (unresolved) effects

such as (substitutional) disorder, model or data errors and/or

over refinement. Atomic sites assigned the wrong scattering type (e.g. Ag

versus Br) should generate 'problem signals' with this test. Data sets

corrected for absorption effects with DELREF techniques (e.g. DIFABS, SHELXA,

XABS2) often show large DELU values for bonds involving the heaviest atom.

Note: The original 'Hirshfeld-test' was defined in absolute terms (see

F.L. Hirshfeld, Acta Cryst. (1976). A32, 239-244).

#---------------------------------------------------------------------------

# >>> Test for unusually high U(eq) as compared with bonded neighbors

_241 .015 0.03 0.05 2 A

Check High      Ueq as Compared to Neighbors for    $A

The U(eq) value of an atom is compared with the average U(eq) for to

non-hydrogen atoms bonded to it. Large differences may indicate that the

wrong atom type was assigned (e.g. N instead of O).

#---------------------------------------------------------------------------

# >>> Test for unusually low  U(eq) as compared with bonded neighbors

_242 .015 0.03 0.05 2 A

Check Low       Ueq as Compared to Neighbors for    $A

The U(eq) value of an atom is compared with the average U(eq) for

non-hydrogen atoms bonded to it. Large differences may indicate that the

wrong atom type was assigned (e.g. N instead of O). False alarms may

occur for terminal groups such as the t-butyl moiety.

#---------------------------------------------------------------------------

# >>> Test for unusually high solvent U(eq) as compared with bonded neighbors

_243 .015 1.00 1.00 4 A

High  'Solvent' Ueq as Compared to Neighbors for    $A

The U(eq) value of an atom in the solvent or ion is compared with the average

U(eq) for non-hydrogen atoms bonded to it. Large differences may indicate

that the wrong atom type was assigned (e.g. N instead of O).

#---------------------------------------------------------------------------

# >>> Test for unusually low solvent  U(eq) as compared with bonded neighbors

_244 .015 1.00 1.00 4 A

Low   'Solvent' Ueq as Compared to Neighbors for    $A

The U(eq) value of an atom in the solvent or ion is compared with the average

U(eq) for non-hydrogen atoms bonded to it. Large differences may indicate

that the wrong atom type was assigned (e.g. N instead of O). False alarms may

occur for terminal groups such as the t-butyl moiety.

#---------------------------------------------------------------------------

# >>> Test for unusually low H-U(eq) as compared with bonded atom

_245 0.01 0.05 0.10 2 A

U(iso) $A Smaller than U(eq) $B by ... $F AngSq

U(iso) of a hydrogen atom is generally expected to be greater than the

U(eq) of the non-hydrogen atom it is attached to.

#---------------------------------------------------------------------------

# >>> Test for unusual anisotropic average UIJ

_250 2.00 4.00 9.00 2 A

Large U3/U1 Ratio for Average U(i,j) Tensor .... $F

An average value of the U(i,j) tensor of the asymmetric unit of a residue

is calculated. An ALERT is generated when the corresponding U3/U1 ratio

deviates significantly from 1.0. Large values of this ratio should be taken

as an indication of possible systematic errors in the data or errors in

the model. Visual inspection of an ORTEP plot will show that many

displacement ellipsoids have their major axis pointing in the same direction.

#===========================================================================

# >>> Test for main residue(s) disorder %

_301    0    0    0 3 A

Note Main Residue  Disorder .................... $F Perc.

Atom sites that are not fully occupied are counted. A large fraction of

disordered atoms may be both a signal for serious structure analysis

problems or less reliable/interesting results.

#---------------------------------------------------------------------------

# >>> Test for (anion/solvent) disorder %

_302    0    0    0 4 A

Note Anion/Solvent Disorder .................... $F Perc.

Atom sites that are not fully occupied are counted. A large fraction of

disordered atoms may be both a signal for serious structure analysis

problems or less reliable/interesting results.

#---------------------------------------------------------------------------

# >>> Test for isolated Hydrogen Atoms

_305    0    0    0 2 A

Isolated Hydrogen Atom (Outside Bond Range ??)      $A

This test reports on hydrogen atoms that are not on bonding distance to any

atom. This ALERT may indicate that the hydrogen atom refined to a

non-bonding position or needs a symmetry operation to bring it to bonding

distance. It also may indicate a problem with incompatible population

parameters (e.g. C - H with population 0.8 and 0.9 repectively).

#---------------------------------------------------------------------------

# >>> Test for isolated Oxygen   Atoms

_306    0    0    0 2 A

Isolated Oxygen Atom (H-atoms Missing ?) .......    $A

This test reports on oxygen atoms that are not within bonding distance to

any other atom in the structure. A common reason may be that no hydrogen

atoms are given for a water molecule. Attempts should be made to locate

those hydrogen atoms from a difference map.

#---------------------------------------------------------------------------

# >>> Test for isolated Metal   Atoms

_307    0    0    0 2 A

Isolated Metal Atom (Unusual !) ................    $A

This test reports on metal atoms that are not bonded or at coordination

distance of other atoms. Isolated ions are very unusual (or non-existent ?)

#---------------------------------------------------------------------------

# >>> Test for single bonded Metal Atoms

_308    0    0    0 2 A

Single Bonded Metal Atom (Unusual !) ...........    $A

This test reports on single bonded (coordinated) metal atoms/ions. This

represents a very unusual situation. There are literature examples where such

a 'single bonded metal' was shown to be a halogen.

#---------------------------------------------------------------------------

# >>> Test for single bonded Oxygen Atoms

_309    0   99   99 2 A

Single Bonded Oxygen (C-O .GT. 1.3 Ang) ........    $A

Single bonded Oxygen with C-O .GT. 1.3 Angstrom. Missing H-Atom ?

#---------------------------------------------------------------------------

# >>> Test for 'too close' (symmetry related) full weight atoms

_310    0    0    0 2 A

$A Deleted (Close to $B)    Dist ..... $F Ang.

This test identifies (very) short contacts between atoms that only becomes

apparent after the application of symmetry on the primary coordinate set.

#---------------------------------------------------------------------------

# >>> Test for isolated Disordered Oxygen   Atoms

_311    0   10   10 2 A

Isolated Disordered Oxygen Atom (No H's ?) .....    $A

This test reports on oxygen atoms (not full weight) that are not within

bonding distance to any other atom in the structure.

A common reason may be that no hydrogen atoms are given for a water molecule.

#---------------------------------------------------------------------------

# >>> Test for C=O-H

_312    0 0.02 0.05 2 A

Strange C-O-H Geometry (C-O .LT. 1.25 Ang) .....    $A

Strange C-O-H geometry with C-O .LT. 1.25 Angstrom detected. Misplaced

H-Atom ?

#---------------------------------------------------------------------------

# >>> Test for O with three covalent bonds

_313    0    2    2 2 A

Oxygen with three covalent bonds (rare) ........    $A

Oxygen atom with three covalent bonds detected. Check for correct atom type

assignment (e.g. N rather than O)

Note: Exceptions are H3O+ (Oximium or Hydroxonium) and

H5O2+ (Hydronium or aqua-hydroxonium) species.

#---------------------------------------------------------------------------

# >>> Check for too many H's on C in C=N bond in main residue(s)

_316    0    0    0 2 A

Too many H on C in C=N Moiety in Main Residue ..    $A

An sp3 hybridized C was detected as part of a C=N moiety. Only one

attached H atom in sp2 configuration is expected and not two.

In SHELXL terms this corresponds with an erroneous AFIX 23 rather than an

AFIX 43 type of H atom position generation and refinement.

#---------------------------------------------------------------------------

# >>> Check for too many H's on C in C=N bond in Solvent/Ion (s)

_317    0    0    0 2 A

Too many H on C in C=N Moiety n Solvent/Ion ....    $A

An sp3 hybridized C was detected as part of a C=N moiety. Only one

attached H atom in sp2 configuration is expected and not two.

In SHELXL terms this corresponds with an erroneous AFIX 23 rather than an

AFIX 43 type of H atom position generation and refinement.

#---------------------------------------------------------------------------

# >>> Hybridisation Problem on N in main residue(s)

_318    0   99   99 2 A

Check Hybridisation of $A in Main Residue .          ?

The test attempts to assign one of three hybridisations to N atoms in main

residue: sp, sp2 or sp3 on the basis of the angles around N.

This ALERT may indicate a misassigned H atom position (e.g. an atom

placed in a sp2 position instead of sp3).

#---------------------------------------------------------------------------

# >>> Hybridisation Problem on N in solvent/ion

_319    0   99   99 2 A

Check Hybridisation of $A in Main Residue .          ?

The test attempts to assign one of three hybridisations to N atoms in main

residue: sp, sp2 or sp3 on the basis of the angles around N.

This ALERT may indicate a misassigned H atom position (e.g. an atom

placed in a sp2 position instead of sp3).

#---------------------------------------------------------------------------

# >>> Hybridisation Problem on C in main residue(s)

_320    0   99   99 2 A

Check Hybridisation of $A in Main Residue .          ?

The test attempts to assign one of three hybridisations to C atoms in main

residue: sp, sp2 or sp3 on the basis of the angles around C. In this way,

missing H atoms or too many H-atoms on a carbon atom should be detected.

#---------------------------------------------------------------------------

# >>> Hybridisation Problem on C in solvent/ion

_321    0   99   99 2 A

Check Hybridisation of $A in Solvent/Ion ..          ?

The test attempts to assign one of three hybridisations to C atoms in

solven/anion: sp, sp2 or sp3 on the basis of the angles around C. In this

way missing H atoms or too many H-atoms on a carbon atom should be detected.

#---------------------------------------------------------------------------

# >>> Hybridisation Problem on non-C in main residue(s)

_322    0   99   99 2 A

Check Hybridisation of $A in Main Residue .          ?

The test attempts to assign one of three hybridisations to a non-C atom in

the main residue: sp, sp2 or sp3 on the basis of the angles around the

non-C atom. In this way, missing H atoms or too many H-atoms should be

detected.

#---------------------------------------------------------------------------

# >>> Hybridisation Problem on non-C in Solvent/Ion

_323    0   99   99 2 A

Check Hybridisation of $A in Solvent/Ion             ?

The test attempts to assign one of three hybridisations to a non-C atom in

the solvent/anion: sp, sp2 or sp3 on the basis of the angles around the

non-C atom. In this way, missing H atoms or too many H-atoms should be

detected.

#---------------------------------------------------------------------------

# >>> Check for possibly missing H on coordinating X-N-X in main residue

_324    0   99   99 2 A

Check for Possibly Missing H on Coordinating....    $A

Check for possibly missing Hydrogen atom on Nitrogen coordinating to

a metal in the main residue.

#---------------------------------------------------------------------------

# >>> Check for possibly missing H on coordinating X-N-X in solvent/anion

_325    0   99   99 2 A

Check for Possibly Missing H on Coordinating....    $A

Check for possibly missing Hydrogen atom on Nitrogen coordinating to

a metal in the solvent/anion.

#---------------------------------------------------------------------------

# >>> Check for possibly missing H on potentially sp3 Carbon

_326    0    0   99 2 A

Check for Possibly Missing H on sp3? Carbon ....    $A

Check for possibly missing Hydrogen atom on Carbon with sp3-like geometry

in the main residue.

#---------------------------------------------------------------------------

# >>> Check for possibly missing H on potentially sp3 Carbon

_327    0    0   99 2 A

Check for Possibly Missing H on sp3? Carbon ....    $A

Check for possibly missing Hydrogen atom on Carbon with sp3-like geometry

in the solvent/anion.

#---------------------------------------------------------------------------

# >>> Check for a possibly missing H on potentially sp3 Phosphorus

_328    0   99   99 4 A

Check for Possibly Missing H on sp3? Phosphorus.    $A

Check for a possibly missing Hydrogen atom on Phosphorus with sp3-like

geometry.

#---------------------------------------------------------------------------

# >>> Check Average Phenyl C-C

_330 0.01 0.02 0.03 2 A

Large Average Phenyl  C-C Dist.  $A-$B $F Ang.

The standard average C-C bond distance in a phenyl ring is 1.395 Angstrom.

The actual average ring distance may be larger than expected due to

systematic errors in the cell dimensions (e.g. use of incorrect

wavelength value for the determination of the cell parameters).

#---------------------------------------------------------------------------

# >>> Check Average Phenyl C-C

_331 0.02 0.03 0.04 2 A

Small Average Phenyl  C-C Dist.  $A-$B $F Ang.

The standard average C-C bond distance in a phenyl ring is 1.395 Angstrom.

The average ring distance may be smaller due to large thermal motion or

incorrect cell dimensions.

#---------------------------------------------------------------------------

# >>> Check Phenyl C-C Range

_332 0.15 0.25 0.35 2 A

Large Phenyl C-C Range           $A-$B $F Ang.

The standard average C-C in a phenyl ring is 1.395 Angstrom. Bond distances

in the ring are expected to vary only slightly due to thermal motion or

substituent effects. Large deviations are likely due to data or model errors.

#---------------------------------------------------------------------------

# >>> Check Average in Multiple Substituted Benzene Type C-C

_333 0.03 0.06 0.09 2 A

Check Large Av C6-Ring C-C Dist. $A-$B $F Ang.

The standard average C-C bond distance in a flat six carbon atom containing

aromatic ring is 1.395 Angstrom.

The actual average ring distance may be larger than expected due to

substituents such as =O, single bonds or systematic errors in the cell

dimensions (E.g. when the wrong wavelength is used in the derivation of

the cell parameters).

#---------------------------------------------------------------------------

# >>> Check Average in Multiple Substituted Benzene Type C-C

_334 0.03 0.06 0.09 2 A

Small Average Benzene C-C Dist.  $A-$B $F Ang.

The standard average C-C bond distance in a benzene ring is 1.395 Angstrom.

The average ring distance may be smaller due to large thermal motion,

substituents such as =O or incorrect cell dimensions.

#---------------------------------------------------------------------------

# >>> Check Multiple Substituted Benzene Type C-C Range

_335 0.15 0.25 0.35 2 A

Check Large C6 Ring C-C Range .  $A-$B $F Ang.

The standard average C-C bond distance in a benzene ring is 1.395 Angstrom.

Bond distances in the ring are expected to vary only slightly when due to

substituent effects (exceptions include =O substituents). Large deviations

may indicate data or model errors.

#---------------------------------------------------------------------------

# >>> Check Average Torsion Angle in cyclo-hexane ring

_338 25.0 45.0 60.0 4 A

Small Average Tau in cyclohexane $A-$B $F Deg.

Cyclohexane moieties should have be significantly puckered as measured by

the average torsion angle tau. Unresolved disorder generally results in

flattened rings and alongated displacement ellipsoids. A disorder model

should be included in the calculations.

#---------------------------------------------------------------------------

# >>> Check Bond Precision for C-C in Light Atom Structures (Z(max) < 20)

_340    4   10   50 3 A

Low Bond Precision on C-C Bonds (x 1000) Ang ... $I

The average su for X-Y bonds is tested (named bond-precision). X-Y will

generally be C-C bonds, unless there are none. In the last case the su's of

the lowest element numbers are considered (excluding hydrogen). There are

three test ranges: one for structures with the largest element Z < 20, one

for the largest Z in the range 20 to 39 and one for structures with Z(max) 40

or higher (_340, _341 and _342 respectively)

#---------------------------------------------------------------------------

# >>> Check Bond Precision for C-C in Structures (19 < Z(max) < 40)

_341    6   15   75 3 A

Low Bond Precision on C-C Bonds (x 1000) Ang ... $I

The average su for X-Y bonds is tested (named bond-precision). X-Y will

generally be C-C bonds, unless there are none. In the last case the su's of

the lowest element numbers are considered (excluding hydrogen). There are

three test ranges: one for structures with the largest element Z < 20, one

for the largest Z in the range 20 to 39 and one for structures with Z(max) 40

or higher (_340, _341 and _342 respectively)

#---------------------------------------------------------------------------

# >>> Check Bond Precision for C-C in Structures (Z(max) > 39)

_342    8   20  100 3 A

Low Bond Precision on C-C Bonds (x 1000) Ang ... $I

The average su for X-Y bonds is tested (named bond-precision). X-Y will

generally be C-C bonds, unless there are none. In the last case the su's of

the lowest element numbers are considered (excluding hydrogen). There are

three test ranges: one for structures with the largest element Z < 20, one

for the largest Z in the range 20 to 39 and one for structures with Z(max) 40

or higher (_340, _341 and _342 respectively)

#---------------------------------------------------------------------------

# >>> Hybridisation Problem on C in main residue(s)

_343    0    0    0 2 A

Check $A Angle Range in Main Residue for ..       $B

The angle range is larger than usual for the tentatively assigned

hybridisation of the reported atom in the main residue.

#---------------------------------------------------------------------------

# >>> Hybridisation Problem on C in solvent/ion

_344    0    0    0 2 A

Check $A Angle Range in Solvent/Ion for ....      $B

The angle range is larger than usual for the tentatively assigned

hybridisation of the reported atom in the solven/anion.

#---------------------------------------------------------------------------

# >>> Test for short C - H (Angstrom Difference) XRAY: 0.96

_350 0.11  0.2  0.3 3 A

Short   C-H Bond (0.96A)  $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc.  Default C-H = 0.96 Ang. (X-Ray) value from SHELXL.

#---------------------------------------------------------------------------

# >>> Test for long  C - H (Angstrom Difference) XRAY: 0.96

_351 0.14  0.2  0.3 3 A

Long    C-H Bond (0.96A)  $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc.  Default C-H = 0.96 Ang. (X-Ray) value from SHELXL.

#---------------------------------------------------------------------------

# >>> Test for short N - H (Angstrom Difference) XRAY: 0.87

_352  0.1  0.2  0.3 3 A

Short   N-H Bond (0.87A)  $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc.  Default N-H = 0.87 Ang. (X-Ray) value from SHELXL.

#---------------------------------------------------------------------------

# >>> Test for long  N - H (Angstrom Difference) XRAY: 0.87

_353 0.13  0.2  0.3 3 A

Long    N-H Bond (0.87A)  $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc.  Default N-H = 0.87 Ang. (X-Ray) value from SHELXL.

#---------------------------------------------------------------------------

# >>> Test for short O - H (Angstrom Difference) XRAY: 0.82

_354  0.1  0.2  0.3 3 A

Short   O-H Bond (0.82A)  $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc.  Default O-H = 0.82 Ang. (X-Ray) value from SHELXL.

#---------------------------------------------------------------------------

# >>> Test for long  O - H (Angstrom Difference) XRAY: 0.82

_355 0.18 0.25  0.3 3 A

Long    O-H Bond (0.82A)  $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc.  Default O-H = 0.82 Ang. (X-Ray) value from SHELXL.

#---------------------------------------------------------------------------

# >>> Test for short C4 - C4 (Angstrom Difference) XRAY: 1.54

_360  0.1  0.2  0.3 2 A

Short  C(sp3)-C(sp3) Bond $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc. Default C4-C4 = 1.54 Ang. (X-Ray) value from

Ladd & Palmer, Structure Determination by Xray Crystallography (1985).

Note:

- C4-C4 indicates a bond between atoms with 4 bonds each.

#---------------------------------------------------------------------------

# >>> Test for long  C4 - C4 (Angstrom Difference) XRAY: 1.54

_361  0.1  0.2  0.3 2 A

Long   C(sp3)-C(sp3) Bond $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc. Default C4-C4 = 1.54 Ang. (X-Ray) value from

Ladd & Palmer, Structure Determination by Xray Crystallography (1985).

Note:

- C4-C4 indicates a bond between atoms with 4 bonds each.

#---------------------------------------------------------------------------

# >>> Test for short C4 - C3 (Angstrom Difference) XRAY: 1.52

_362  0.1  0.2  0.3 2 A

Short  C(sp3)-C(sp2) Bond $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc. Default C4-C3 = 1.52 Ang. (X-Ray) value from

Ladd & Palmer, Structure Determination by Xray Crystallography (1985).

Note:

- C4-C3 indicates a bond between an atom with 4 bonds and one with 3 bonds.

- Conjugated systems may cause some 'false alarm' messages.

#---------------------------------------------------------------------------

# >>> Test for long  C4 - C3 (Angstrom Difference) XRAY: 1.52

_363  0.1  0.2  0.3 2 A

Long   C(sp3)-C(sp2) Bond $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc. Default C4-C3 = 1.52 Ang. (X-Ray) value from

Ladd & Palmer, Structure Determination by Xray Crystallography (1985).

Note:

- C4-C3 indicates a bond between an atom with 4 bonds and one with 3 bonds.

- Conjugated systems may cause some 'false alarm' messages.

#---------------------------------------------------------------------------

# >>> Test for short C4 - C2 (Angstrom Difference) XRAY: 1.46

_364  0.1  0.2  0.3 2 A

Short  C(sp3)-C(sp)  Bond $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc. Default C4-C2 = 1.46 Ang. (X-Ray) value from

Ladd & Palmer, Structure Determination by Xray Crystallography (1985).

Note:

- C4-C2 indicates a bond between an atom with 4 bonds and one with 2 bonds.

- Conjugated systems may cause some 'false alarm' messages.

#---------------------------------------------------------------------------

# >>> Test for long  C4 - C2 (Angstrom Difference) XRAY: 1.46

_365  0.1  0.2  0.3 2 A

Long   C(sp3)-C(sp)  Bond $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc. Default C4-C2 = 1.46 Ang. (X-Ray) value from

Ladd & Palmer, Structure Determination by Xray Crystallography (1985).

Note:

- C4-C2 indicates a bond between an atom with 4 bonds and one with 2 bonds.

- Conjugated systems may cause some 'false alarm' messages.

#---------------------------------------------------------------------------

# >>> Test for short C? - C? (Angstrom Difference) XRAY: 1.50

_366  0.1  0.2  0.3 2 A

Short? C(sp?)-C(sp?) Bond $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc. Default C?-C? = 1.50 Ang. (X-Ray) value from

Ladd & Palmer, Structure Determination by Xray Crystallography (1985).

This ALERT may also arise when the hybridisation at least one atom is

not recognized.

#---------------------------------------------------------------------------

# >>> Test for long  C? - C? (Angstrom Difference) XRAY: 1.50

_367  0.1  0.2  0.3 2 A

Long?  C(sp?)-C(sp?) Bond $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc. Default C?-C? = 1.50 Ang. (X-Ray) value from

Ladd & Palmer, Structure Determination by Xray Crystallography (1985).

#---------------------------------------------------------------------------

# >>> Test for short C3 - C3 (Angstrom Difference) XRAY: 1.34

_368  0.1  0.2  0.3 2 A

Short  C(sp2)-C(sp2) Bond $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc. Default C3-C3 = 1.34 Ang. (X-Ray) value from

Ladd & Palmer, Structure Determination by Xray Crystallography (1985).

Note:

- C3-C3 indicates a bond between atoms with 3 bonds each.

- Conjugated systems may cause some 'false alarm' messages.

#---------------------------------------------------------------------------

# >>> Test for long  C3 - C3 (Angstrom Difference) XRAY: 1.34

_369 0.18 0.22  0.3 2 A

Long   C(sp2)-C(sp2) Bond $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc.  Default C3-C3 = 1.34 Ang. (X-Ray) value from

Ladd & Palmer, Structure Determination by Xray Crystallography (1985).

Note:

- C3-C3 indicates a bond between atoms with 4 with 3 bonds each.

- Conjugated systems may cause some 'false alarm' messages.

- A notable exception is the C-C bond in -C(=O)-C(=O)- systems with an

observed mean value of 1.54 Angstrom.

#---------------------------------------------------------------------------

# >>> Test for short C3 - C2 (Angstrom Difference) XRAY: 1.31

_370  0.1  0.2  0.3 2 A

Short  C(sp2)-C(sp1) Bond $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc. Default C3-C2 = 1.31 Ang. (X-Ray) value from

Ladd & Palmer, Structure Determination by Xray Crystallography (1985).

Note:

- C3-C2 indicates a bond between an atom with 3 bonds and one with 2 bonds.

- Conjugated systems may cause some 'false alarm' messages.

#---------------------------------------------------------------------------

# >>> Test for long  C3 - C2 (Angstrom Difference) XRAY: 1.31

_371 0.1  0.2   0.3 2 A

Long   C(sp2)-C(sp1) Bond $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc. Default C3-C2 = 1.31 Ang. (X-Ray) value from

Ladd & Palmer, Structure Determination by Xray Crystallography (1985).

Note:

- C3-C2 indicates a bond between an atom with 3 bonds and one with 2 bonds.

- Conjugated systems may cause some 'false alarm' messages.

#---------------------------------------------------------------------------

# >>> Test for short C2 - C2 (Angstrom Difference) XRAY: 1.25

_372  0.1  0.2  0.3 2 A

Short  C(sp)-C(sp)   Bond $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc. Default C2-C2 = 1.25 Ang. (X-Ray) value from

Ladd & Palmer, Structure Determination by Xray Crystallography (1985).

Note:

- C2-C2 indicates a bond between atoms with 2 bonds each.

- Conjugated systems may cause some 'false alarm' messages.

#---------------------------------------------------------------------------

# >>> Test for long  C2 - C2 (Angstrom Difference) XRAY: 1.25

_373  0.1  0.2  0.3 2 A

Long   C(sp)-C(sp)   Bond $A -  $B ... $F Ang.

Large deviations from generally accepted values may indicate model problems,

over refinement etc. Default C2-C2 = 1.25 Ang. (X-Ray) value from

Ladd & Palmer, Structure Determination by Xray Crystallography (1985).

Note:

- C2-C2 indicates a bond between atoms with 2 bonds each.

- Conjugated systems may cause some 'false alarm' messages.

#---------------------------------------------------------------------------

# >>> Test for long  N - N Bond  (> 1.45 Angstrom)

_374  0.0  0.0  0.0 2 A

Long   N - N         Bond $A -  $B ... $F Ang.

Large deviations from generally observed bond distances may indicate model

problems, over-refinement etc. Check for wrong atom-type assignments.

For an example see: Acta Cryst. (2003) E59, m710-m712.

#---------------------------------------------------------------------------

# >>> Test for incorrectly Oriented Methyl Moiety

_380  0.0 10.0 10.0 4 A

Check Incorrectly? Oriented X(sp2)-Methyl Moiety    $A

This test alerts for possible incorrectly oriented CH3 moieties.

(E.g. AFIX 33 instead of AFIX 137 etc. within the SHELXL realm)

#---------------------------------------------------------------------------

# >>> Test Methyl Moiety X-C-H Bond Angle

_390  6.5  9.5 14.5 3 A

Deviating Methyl $A X-C-H Bond Angle ...... $F Deg.

Unusual Methyl Moiety X-C-H Angle (Ideally 109 Degrees).

#===========================================================================

# >>> Test Methyl Moiety H-C-H Bond Angle

_391  7.5  9.5 14.5 3 A

Deviating Methyl $A H-C-H Bond Angle ...... $F Deg.

Unusual Methyl Moiety H-C-H Angle (ideally 109 Degrees).

#===========================================================================

# >>> Test X-O-Y Angle

_395 20.0 40.0 70.0 2 A

Deviating  X-O-Y  Angle from 120 Deg for $A $F Deg.

The X-O-Y angle is significantly larger than the expected 120.0 Degrees.

#===========================================================================

# >>> Test Si-O-Si Angle

_396 10.0 20.0 30.0 2 A

Deviating Si-O-Si Angle from 150 Deg for $A $F Deg.

The Si-O-Si angle is significantly larger than the expected 150.0 Degrees.

#===========================================================================

# >>> Test for short non-bonding intra H..H contacts

_410  0.4  0.5  0.6 2 A

Short Intra H...H Contact  $A..  $B .. $F Ang.

Short intramolecular contacts may arise when H-atoms are in (false)

calculated positions. Short intramolecular contacts may also be a sign for a

false structure with the molecule sitting on a site with improper site

symmetry (e.g. '2' instead of '-1') which may happen when a lattice

translation is missed. Short contacts are defined using a van der Waals

radius of 1.2 Angstrom. For intramolecular contacts alerts are generated

for contacts less than 2.0 Angstrom.

#===========================================================================

# >>> Test for short non-bonding inter H..H contacts

_411    0  0.4  0.6 2 A

Short Inter H...H Contact  $A..  $B .. $F Ang.

Short intermolecular H..H contacts may indicate incorrectly determined

structures (i.e. wrong symmetry, missed translation symmetry, wrong position

with reference to the symmetry elements, hydrogen atoms on atoms where there

should not be any  etc..)

Short intermolecular contacts may be indicative for inconsistent symmetry

data (e.g. coordinates for space-group P43 and symmetry specified as P41 or

P21/n & P21/c confusions). Short contacts are defined using a van der Waals

radius of 1.2 Angstrom. For intermolecular contacts, an alert is generated

for contacts less than 2.4 Angstrom.

#===========================================================================

# >>> Test for short non-bonding intra H..H contacts (involving XH3)

_412  0.5  0.6  0.7 2 A

Short Intra XH3 .. XHn     $A..  $B .. $F Ang.

Short intramolecular contacts may arise when H-atoms are in (false)

calculated positions. Short intramolecular contacts may also be a sign for a

false structure with the molecule sitting on a site with improper site

symmetry (e.g. '2' instead of '-1') which may happen when a lattice

translation is missed. Short contacts are defined using a van der Waals

radius of 1.2 Angstrom. Short H .. H contact involving CH3 H-atoms are often

hampered by the fact that they involve H atoms at not optimal calculated

positions.

#===========================================================================

# >>> Test for short non-bonding inter H..H contacts (involving XH3)

_413  0.0  0.3  0.5 2 A

Short Inter XH3 .. XHn     $A..  $B .. $F Ang.

Short intermolecular H..H contacts may indicate incorrectly determined

structures (i.e. wrong symmetry, missed translation symmetry, wrong position

with reference to the symmetry elements, hydrogen atoms on atoms where there

should not be any  etc..). Short intermolecular contacts may be indicative

for inconsistent symmetry data (e.g. coordinates for space-group P43 and

symmetry specified as P41 or P21/n & P21/c confusions). Short contacts are

defined using a van der Waals radius of 1.2 Angstrom. Short H .. H contact

involving CH3 H-atoms are often hampered by the fact that they involve

H atoms at not optimal calculated positions.

#===========================================================================

# >>> Test for short non-bonding intra D-H..H-X contacts

_414  0.4  0.5  0.6 2 A

Short Intra D-H..H-X       $A..  $B .. $F Ang.

Short non-bonding intra D-H..H-X contact.

#===========================================================================

# >>> Test for short non-bonding inter D-H..H-X contacts

_415  0.1  0.3  0.5 2 A

Short Inter D-H..H-X       $A..  $B .. $F Ang.

Short non-bonding inter D-H..H-X contact.

#===========================================================================

# >>> Test for short non-bonding intra D-H..H-D contacts

_416  0.4  0.5  2.5 2 A

Short Intra D-H..H-D       $A..  $B .. $F Ang.

Short non-bonding intra D-H..H-D contacts may be related to disordered or

misplaced H-atoms.

#===========================================================================

# >>> Test for short non-bonding inter D-H..H-D contacts

_417    0  0.3  2.5 2 A

Short Inter D-H..H-D       $A..  $B .. $F Ang.

Short non-bonding inter D-H..H-D contacts may be related to disordered or

misplaced H-atoms.

#===========================================================================

# >>> Test for D-H without acceptor

_420    0  1.5   99 2 A

D-H Without Acceptor      $A -  $B ...          ?

Potential hydrogen bond donors are checked for the presence of suitable

acceptors using commonly used (Jeffrey) H-bond criteria. As a general rule

there should be an acceptor for each donor. Exceptions are very rare for O-H

and more common for -NH and -NH2. A common error is an -OH on a calculated

position pointing in the wrong direction.

#===========================================================================

# >>> Test for short non-bonding inter D...A contacts

_430    0 0.19 0.50 2 A

Short Inter D...A Contact  $A..  $B .. $F Ang.

This test alerts for possibly missed Hydrogen bonds as indicated by short

(i.e. shorter than sum of the van der Waals radii - 0.2 Angstrom)

Donor - Acceptor distances.

Note: Short C=O .. O=C are observed sometimes when part of three-centre

O-H, N-H or C-H O..O bridging.

#===========================================================================

# >>> Test for short non-bonding inter HL...A contacts

_431 0.10 0.30 0.50 2 A

Short Inter HL..A Contact  $A..  $B .. $F Ang.

This test reports on short intermolecular Halogen .. Donor/Acceptor

atom-type distances.

#===========================================================================

# >>> Test for short non-bonding inter X...Y contacts

_432  0.2 0.30 0.50 2 A

Short Inter X...Y Contact  $A..  $B .. $F Ang.

This test raised an ALERT for short intermolecular contacts.

In general, intermolecular contact distances should be not much smaller than

the sum of the associated van der Waals Radii. More often than not, such

short contacts can be a warning sign for errors. All short contacts should

therefore be examined in some detail. Interesting exceptions are carbonyl-

carbonyl interactions that often feature short O...C contacts (see Allen

et al. (1998) B54, 320-329, short NO2 O...O interactions and BF4(-) to

(aromatic) carbon contacts.

#===========================================================================

# >>> Test for short non-bonding minor..minor inter X...Y contacts

_433  0.5 0.85 1.00 4 A

Short Inter X...Y Contact  $A..  $B .. $F Ang.

This test raised an ALERT for short intermolecular contacts between minor

disorder components. In general, intermolecular contact distances should be

not much smaller than the sum of the associated van der Waals Radii. More

often than not, such short contacts can be a warning sign for errors. All

short contacts should therefore examined in some detail. Interesting

exceptions are carbonyl-carbonyl interactions that often feature short O...C

contacts (see Allen et al. (1998) B54, 320-329.

#===========================================================================

# >>> Test for too large H...A

_480 -.12 0.25  0.5 4 A

Long H...A H-Bond Reported $A..  $B .. $F Ang.

Check this (unrealistically) long reported H..A contact.

Jeffrey criterium: Contact .LT. vdWR(H) + vdWR(A) - 0.12 Angstrom.

#===========================================================================

# >>> Test for too large D...A

_481  0.5  1.0  2.0 4 A

Long D...A H-Bond Reported $A..  $B .. $F Ang.

Check this (unrealistically) long reported D..A contact.

Jeffrey criterium: Contact .LT. vdWR(D) + vdWR(A) + 0.50 Angstrom.

#===========================================================================

# >>> Test for too small D-H...A Angle

_482  80. 90.0 100. 4 A

Small D-H..A Angle Rep for $A..  $B .. $F Deg.

Check this unrealistically small reported D-H..A Angle.

Jeffrey criterium: D-H..A Angle .GT. 100 degrees.

#===========================================================================

# >>> Test for not rounded D-H...A Angle

_484    0    2    2 4 I

Round D-H..A Angle Rep for $A..  $B to $I Deg.

D-H..A angles without s.u. should be rounded to integer values for

publication purposes (Required for Acta Cryst.).

#===========================================================================

# >>> Test for (Unreported) solvent accessible voids

_601   30  100  200 2 A

Structure Contains Solvent Accessible VOIDS of . $F A**3

Crystal structures in general do not contain large solvent accessible voids

in the lattice. Most structures lose their long-range ordering when solvent

molecules leave the crystal. Only when the remaining network is strongly

bonded (e.g. zeolites and some hydrogen bonded networks) the crystal

structure may survive.

Residual voids in a structure may indicate the omission of (disordered)

density from the model. Disordered density may go undetected when smeared

since peak search programs are not designed to locate maxima on density

ridges. The presence or absence of residual density in the void may be

veryfied on a printed/plotted difference Fourier map or with PLATON/SQUEEZE.

Voids of 40 Ang**3 may accommodate H2O. Small molecules such as

Tetrahydrofuran have typical volumes in the 100 to 200 Ang**3 range.

This test reports the volume of the largest solvent accessible void in the

structure.

A paper reporting a crystal structure with a significant solvent accessible

void should at the least discuss the issue.

#===========================================================================

# >>> Test for TOO LARGE (Unreported) solvent accessible voids

_602    0    0    0 2 A

VERY LARGE Solvent Accessible VOID(S) in Structure        !

This test reports on a solvent accessible void in the structure, too large

or too time consuming for the current PLATON version for a more detailed

analysis as part of the validation run. Use the SOLV option for more details.

Such a warning might also indicate that the symmetry is incomplete

e.g. should have been specified as P-1 and not P1, leaving out half of

the unit cell content.

#===========================================================================

# >>> Test for TOO LARGE Unit Cell for VOID search

_603    0    0    0 4 A

Unit Cell TOO large for VOID SEARCH in Structure          !

No search for solvent accessible VOIDS done as part of VALIDATION in

view of large unitcell.

#===========================================================================

# >>> Test for TOO Many VOIDS

_604    0    0    0 4 A

Too Many VOIDS Detected in Structure .............        !

Too many solvent accessible VOIDS.

# ===================================================================

# >>> Test for (Reported)  solvent accessible voids

_605    0    0    0 4 A

Structure Contains Solvent Accessible VOIDS of . $F A**3

Crystal structures in general do not contain large solvent accessible voids

in the lattice. Most structures lose their long-range ordering when solvent

molecules leave the crystal. Only when the remaining network is strongly

bonded (e.g. zeolites and some hydrogen bonded networks) the crystal

structure may survive.

Residual voids in a structure may indicate the omission of (disordered)

density from the model. Disordered density may go undetected when smeared

since peak search programs are not designed to locate maxima on density

ridges. The presence or absence of residual density in the void may be

veryfied on a printed/plotted difference Fourier map or with PLATON/SQUEEZE.

Voids of 40 Ang**3 may accommodate H2O. Small molecules such as

Tetrahydrofuran have typical volumes in the 100 to 200 Ang**3 range.

This test reports the volume of the largest solvent accessible void in the

structure.

A paper reporting a crystal structure with a significant solvent accessible

void should at the least discuss the issue.

Note: The use of PLATON/SQUEEZE was reported in the CIF

#===========================================================================

# >>> Test for TOO LARGE (Reported) solvent accessible voids

_606    0    0    0 4 A

VERY LARGE Solvent Accessible VOID(S) in Structure        !

This test reports on a solvent accessible void in the structure, too large

or too time consuming for the current PLATON version for a more detailed

analysis as part of the validation run. Use the SOLV option for more details.

Such a warning might also indicate that the symmetry is incomplete

e.g. should have been specified as P-1 and not P1, leaving out half of

the unit cell content.

#===========================================================================

# >>> Test for consistency of Bonds and Coordinates in CIF

_701  1.0  2.0  3.0 1 A

Bond    Calc$X, Rep$Y, Dev.. $F Sigma

Bond distances given in the CIF are cross-checked with corresponding

values calculated from the coordinates.  Alerts are set at 1,2 and 3 sigma

deviation levels.

Note: Default s.u.'s are used where no su given (e.g. for C-H)

In general, all differences should be within the associated s.u.

Small differences may arise from rounding.

Very large deviation (or zero distance) normally indicate incorrectly

specified symmetry operations on the associated atoms, or 'cut-and-pasting'

of incompatible CIF's.

#===========================================================================

# >>> Test for consistency of Angles and Coordinates in CIF

_702  1.0  2.0  3.0 1 A

Angle   Calc$X, Rep$Y, Dev.. $F Sigma

Bond Angles given in the CIF are cross-checked with corresponding values

calculated from the coordinates.  Alerts are set at 1,2 and 3 sigma

deviation levels.

In general, all differences should be within the associated s.u.

Small differences may arise from rounding. Very large deviations normally

indicate incorrectly specified symmetry operations on the associated atoms,

or 'cut-and-pasting' of incompatible CIF's.

#===========================================================================

# >>> Test for consistency of Torsions and Coordinates in CIF

_703  1.0  2.0  3.0 1 A

Torsion Calc$X, Rep$Y, Dev.. $F Sigma

Torsion angles given in the CIF are cross-checked with corresponding values

calculated from the coordinates.  Alerts are set at 1,2 and 3 sigma

deviation levels.

In general, all differences should be within the associated s.u.

Small differences may arise from rounding. Very large deviations normally

indicate incorrectly specified symmetry operations on the associated atoms,

or 'cut-and-pasting' of incompatible CIF's.

#===========================================================================

# >>> Test for consistency of Contact Distances and Coordinates in CIF

_704  1.0  2.0  3.0 1 A

Contact Calc$X, Rep$Y, Dev.. $F Sigma

Intermolecular contacts listed in the CIF are checked against the

coordinates in the CIF.

Alerts are set at 1,2 and 3 sigma deviation levels.

#===========================================================================

# >>> Test for consistency of H-Bond D-H distances and Coordinates in CIF

_705  1.0  2.0  3.0 1 A

D-H     Calc$X, Rep$Y, Dev.. $F Sigma

Hydrogen-Bond D-H listed in the CIF is checked.  Alerts are set at

1,2 and 3 sigma deviation levels.

#===========================================================================

# >>> Test for consistency of H-Bond H..A Distances and Coordinates in CIF

_706  1.0  2.0  3.0 1 A

H...A   Calc$X, Rep$Y, Dev.. $F Sigma

Hydrogen-Bond H..A listed in the CIF is checked. Alerts are set at 1,2 and

3 sigma deviation levels.

This ALERT is generally related to incorrect symmetry codes. The symmetry

number s in the symmetry code s_pqr should correspond to the expression

for s in the CIF. Those expressions can be different for different

software packages. E.g. pasting H-bond table data generated with PLATON

into a CIF generated with SHELXL may raise this ALERT. Manual correction

of the symmetry code should be trivial.

#===========================================================================

# >>> Test for consistency of H-Bond D..A Distances and Coordinates in CIF

_707  1.0  2.0  3.0 1 A

D...A   Calc$X, Rep$Y, Dev.. $F Sigma

Hydrogen-Bond D..A listed in the CIF is checked. Alerts are set at 1,2 and

3 sigma deviation levels.

This ALERT is generally related to incorrect symmetry codes. The symmetry

number s in the symmetry code s_pqr should correspond to the expression

for s in the CIF. Those expressions can be different for different

software packages. E.g. pasting H-bond table data generated with PLATON

into a CIF generated with SHELXL may raise this ALERT. Manual correction

of the symmetry code should be trivial.

#===========================================================================

# >>> Test for consistency of H-Bond D-H..A Angles and Coordinates in CIF

_708  1.0  2.0  3.0 1 A

D-H..A  Calc$X, Rep$Y, Dev.. $F Sigma

Hydrogen-Bond Angle D-H..A listed in the CIF is checked. Alerts are set at

1,2 and 3 sigma deviation levels.

This ALERT is generally related to incorrect symmetry codes. The symmetry

number s in the symmetry code s_pqr should correspond to the expression

for s in the CIF. Those expressions can be different for different

software packages. E.g. pasting H-bond table data generated with PLATON

into a CIF generated with SHELXL may raise this ALERT. Manual correction

of the symmetry code should be trivial.

#===========================================================================

# >>> Test for Linear Torsions in CIF

_710    0    0    0 4 I

Delete 1-2-3 or 2-3-4 Linear Torsion Angle ... #    $A

Torsion angles specified in the CIF are checked for the

'linear variety' where one or both of the 1-2-3 and 2-3-4 bond

angles are close to 180 Deg. SHELXL97 will generate those 'torsions' for

molecules containing linear moieties (E.g. Metal-C=O).

#===========================================================================

# >>> Test for label problems for Bonds in CIF

_711    0   99   99 1 A

BOND    Unknown or Inconsistent Label ..........    $A

When labels are found on geometry items (bonds, angles etc.) that are not in

the coordinate list, and alert _71n is issued, related to alert _70n.

#===========================================================================

# >>> Test for label problem for Angles in CIF

_712    0   99   99 1 A

ANGLE   Unknown or Inconsistent Label ..........    $A

When labels are found on geometry items (bonds, angles etc.) that are not in

the coordinate list, and alert _71n is issued, related to alert _70n.

#===========================================================================

# >>> Test for label problem for Torsions in CIF

_713    0   99   99 1 A

TORSION Unknown or Inconsistent Label ..........    $A

When labels are found on geometry items (bonds, angles etc.) that are not in

the coordinate list, and alert _71n is issued, related to alert _70n.

#===========================================================================

# >>> Test for label problem for Contact Distances in CIF

_714    0   99   99 1 A

CONTACT Unknown or Inconsistent Label ..........    $A

When labels are found on geometry items (bonds, angles etc.) that are not in

the coordinate list, and alert _71n is issued, related to alert _70n.

#===========================================================================

# >>> Test for label problem for H-Bond D-H distances in CIF

_715    0   99   99 1 A

D-H     Unknown or Inconsistent Label ..........    $A

When labels are found on geometry items (bonds, angles etc.) that are not in

the coordinate list, and alert _71n is issued, related to alert _70n.

#===========================================================================

# >>> Test for label problem for H-Bond H..A Distances in CIF

_716    0   99   99 1 A

H...A   Unknown or Inconsistent Label ..........    $A

When labels are found on geometry items (bonds, angles etc.) that are not in

the coordinate list, and alert _71n is issued, related to alert _70n.

#===========================================================================

# >>> Test for label problem for  H-Bond D..A Distances in CIF

_717    0   99   99 1 A

D...A   Unknown or Inconsistent Label ..........    $A

When labels are found on geometry items (bonds, angles etc.) that are not in

the coordinate list, and alert _71n is issued, related to alert _70n.

#===========================================================================

# >>> Test for label problem for H-Bond D-H..A Angles in CIF

_718    0   99   99 1 A

D-H..A  Unknown or Inconsistent Label ..........    $A

When labels are found on geometry items (bonds, angles etc.) that are not in

the coordinate list, and alert _71n is issued, related to alert _70n.

#===========================================================================

# >>> Test for Unusual Labels

_720    0    2    2 4 I

Number of Unusual/Non-Standard Labels .......... $I

Up to 4 Character Labels of the type C11, H101, N10A, i.e. chemical symbol +

number + optional letter are to be preferred.

#===========================================================================

# >>> Test for consistency of Bonds and Coordinates in CIF

_721  1.0  2.0  3.0 1 A

Bond    Calc$X, Rep$Y Dev... $F Ang.

Same as 701 but for distance without su (esd). Difference is tested in terms

of Angstroms.

#===========================================================================

# >>> Test for consistency of Angles and Coordinates in CIF

_722  1.0  2.0  3.0 1 A

Angle   Calc$X, Rep$Y Dev... $F Deg.

Same as 702 but for angle without su (esd). Difference is tested in terms of

Degrees.

#===========================================================================

# >>> Test for consistency of Torsions and Coordinates in CIF

_723  1.0  2.0  3.0 1 A

Torsion Calc$X, Rep$Y Dev... $F Sigma

Same as 703 but for torsion without su (esd). Difference is tested in terms

of Degrees.

#===========================================================================

# >>> Test for consistency of Contact Distances and Coordinates in CIF

_724  1.0  2.0  3.0 1 A

Contact Calc$X, Rep$Y Dev... $F Ang.

Same as 704, but for distance without su (esd). Difference is tested in

terms of Angstroms.

#===========================================================================

# >>> Test for consistency of H-Bond D-H distances and Coordinates in CIF

_725  1.0  2.0  3.0 1 A

D-H     Calc$X, Rep$Y Dev... $F Ang.

Same as 705 but for distance without s.u. (esd). Differences are tested in

terms of Angstrom.

#===========================================================================

# >>> Test for consistency of H-Bond H..A Distances and Coordinates in CIF

_726  1.0  2.0  3.0 1 A

H...A   Calc$X, Rep$Y Dev... $F Ang.

Same as 706 but for distance without s.u. (esd). Differences are tested in

terms of Angstrom.

#===========================================================================

# >>> Test for consistency of H-Bond D..A Distances and Coordinates in CIF

_727  1.0  2.0  3.0 1 A

D...A   Calc$X, Rep$Y Dev... $F Ang.

Same as 707 but for distance without s.u. (esd). Differences are tested in

terms of Angstrom.

#===========================================================================

# >>> Test for consistency of H-Bond D-H..A Angles and Coordinates in CIF

_728  1.0  2.0  3.0 1 A

D-H..A  Calc$X, Rep$Y Dev... $F Deg.

Same as 708 but for angle without s.u. (esd). Differences are tested in

terms of Degrees.

#===========================================================================

# >>> Test for consistency of Bond su's and Coordinate su's in CIF

_731  2.0  4.0  8.0 1 A

Bond    Calc$X, Rep$Y ...... $F su-Ratio

A large ratio of the reported and calculated bond s.u.'s is found.

The use of a DFIX instruction might cause such a warning since

calculated s.u.'s are based on reported variances only.

Note_1: su's on the unitcell dimensions are taken into account in the

calculation of expected su's. This may result in large differences

between expected and reported su's when this contribution is not included

in the reported su's, in particular for inaccurate unitcells.

Note_2: Another source for the discrepancy between calculated and

reported su's can be that the validation software has access only to

the variances of the refined parameters as opposed to the full co-variance

matrix used by e.g. SHELXL for the calculation of derived parameters

with associated su's. Constrained/restrained refinement may cause largei

co-variances.

#===========================================================================

# >>> Test for consistency of Angles and Coordinates in CIF s.u.'s

_732  2.0  4.0  8.0 1 A

Angle   Calc$X, Rep$Y ...... $F su-Ratio

A large ratio of the reported and calculated bond angle s.u.'s is found.

This check should warn for erroneous rounding: E.g. 105.5(19) to 105.5(2) or

105.0(5) to 105(5) etc. Note: Large differences are possible when certain

constraints/restraints were applied in the refinement (e.g. the FLAT option

in SHELXL97).

Note: su's on the unitcell dimensions are taken into account in the

calculation of expected su's. This may result in large differences

between expected and reported su's when this contribution is not included

in the reported su's, in particular for inaccurate unitcells.

Note_2: Another source for the discrepancy between calculated and

reported su's can be that the validation software has access only to

the variances of the refined parameters as opposed to the full co-variance

matrix used by e.g. SHELXL for the calculation of derived parameters

with associated su's. Constrained/restrained refinement may cause largei

co-variances.

#===========================================================================

# >>> Test for consistency of Torsions and Coordinates in CIF  s.u's

_733  4.0  6.0  8.0 1 A

Torsion Calc$X, Rep$Y ...... $F su-Ratio

A large ratio of the reported and calculated torsion angle s.u.'s is found.

This check should warn for erroneous rounding: E.g. 105.5(19) to 105.5(2)

or 105.0(5) to 105(5) etc.

Note: su's on the unitcell dimensions are taken into account in the

calculation of expected su's. This may result in large differences

between expected and reported su's when this contribution is not included

in the reported su's, in particular for inaccurate unitcells.

Note_2: Another source for the discrepancy between calculated and

reported su's can be that the validation software has access only to

the variances of the refined parameters as opposed to the full co-variance

matrix used by e.g. SHELXL for the calculation of derived parameters

with associated su's. Constrained/restrained refinement may cause largei

co-variances.

#===========================================================================

# >>> Test for consistency of Contact Distance s.u. and Coordinate s.u. in CIF

_734  2.0  4.0  8.0 1 A

Contact Calc$X, Rep$Y ...... $F su-Ratio

A large ratio of the reported and calculated contact distance s.u.'s is found.

Note: su's on the unitcell dimensions are taken into account in the

calculation of expected su's. This may result in large differences

between expected and reported su's when this contribution is not included

in the reported su's, in particular for inaccurate unitcells.

#===========================================================================

# >>> Test for consistency of H-Bond D-H distance s.u. and Coordinate s.u in CIF

_735  2.0  4.0  8.0 1 A

D-H     Calc$X, Rep$Y ...... $F su-Ratio

A large ratio of the reported and calculated H-bond D-H distance s.u.'s is

found. The use of a DFIX instruction might cause such a warning since

calculated s.u.'s are based on reported variances only.

Note: su's on the unitcell dimensions are taken into account in the

calculation of expected su's. This may result in large differences

between expected and reported su's when this contribution is not included

in the reported su's, in particular for inaccurate unitcells.

#===========================================================================

# >>> Test for consistency of H-Bond H..A Distance s.u. and Coordinates in CIF

_736  2.0  4.0  8.0 1 A

H...A   Calc$X, Rep$Y ...... $F su-Ratio

A large ratio of the reported and calculated H-bond H..A distance s.u.'s

is found.

Note: su's on the unitcell dimensions are taken into account in the

calculation of expected su's. This may result in large differences

between expected and reported su's when this contribution is not included

in the reported su's, in particular for inaccurate unitcells.

#===========================================================================

# >>> Test for consistency of H-Bond D..A Distance s.u. and Coordinates in CIF

_737  2.0  4.0  8.0 1 A

D...A   Calc$X, Rep$Y ...... $F su-Ratio

A large ratio of the reported and calculated H-Bond D...A distance s.u.'s is

found.

#===========================================================================

# >>> Test for consistency of H-Bond D-H..A Angle and Coordinates in CIF s.u.

_738  2.0  4.0  8.0 1 A

D-H..A  Calc$X, Rep$Y ...... $F su-Ratio

A large ratio of the reported and calculated H-Bond D-H..A angle s.u.'s

is found.

Note: su's on the unitcell dimensions are taken into account in the

calculation of expected su's. This may result in large differences

between expected and reported su's when this contribution is not included

in the reported su's, in particular for inaccurate unitcells.

#===========================================================================

# >>> Test for missing Bond su in CIF

_741  0.0 99.0 99.0 1 A

Bond    Calc$X, Rep$Y ......    Missing su

Likely missing s.u. on Bond in CIF.

#===========================================================================

# >>> Test for missing Angle s.u. in CIF

_742  0.0 99.0 99.0 1 A

Angle   Calc$X, Rep$Y ......    Missing su

Likely missing s.u. on Bond angle in CIF.

#===========================================================================

# >>> Test for missing Torsion s.u. in CIF

_743  0.0 99.0 99.0 1 A

Torsion Calc$X, Rep$Y ......    Missing su

Likely missing s.u. on Torsion angle in CIF.

#===========================================================================

# >>> Test for missing Contact Distance s.u.  in CIF

_744  0.0 99.0 99.0 1 A

Contact Calc$X, Rep$Y ......    Missing su

Likely missing s.u. on contact Distance in CIF.

#===========================================================================

# >>> Test for missing H-Bond D-H distance s.u. in CIF

_745  0.0 99.0 99.0 1 A

D-H     Calc$X, Rep$Y ......    Missing su

Likely missing s.u. on H-Bond D-H distance in CIF.

#===========================================================================

# >>> Test for missing H-Bond H..A Distance s.u. in CIF

_746  0.0 99.0 99.0 1 A

H...A   Calc$X, Rep$Y ......    Missing su

Likely missing s.u. on H-Bond H...A distance in CIF.

#===========================================================================

# >>> Test for missing H-Bond D..A Distance s.u. in CIF

_747  0.0 99.0 99.0 1 A

D...A   Calc$X, Rep$Y ......    Missing su

Likely missing s.u. on H-Bond D...A distance in CIF.

#===========================================================================

# >>> Test for missing H-Bond D-H..A Angle s.u. in CIF

_748  0.0 99.0 99.0 1 A

D-H..A  Calc$X, Rep$Y ......    Missing su

Likely missing s.u. on H-Bond D-H..A angle in CIF.

#===========================================================================

# >>> Test for senseless Bond s.u. in CIF

_751  0.0 99.0 99.0 4 A

Bond    Calc$X, Rep$Y ......  Senseless su

An s.u. should not be given in the CIF for constrained distances.

Please check for proper refinement status flags (e.g. R)

#===========================================================================

# >>> Test for senseless Angle s.u. in CIF

_752  0.0 99.0 99.0 4 A

Angle   Calc$X, Rep$Y ......  Senseless su

An s.u. should not be given in the CIF for constrained angles.

Please check for proper refinement status flags (e.g. R)

#===========================================================================

# >>> Test for senseless Torsion s.u. in CIF

_753  0.0 99.0 99.0 4 A

Torsion Calc$X, Rep$Y ......  Senseless su

An s.u. should not be given in the CIF for constrained torsion angles.

Please check for proper refinement status flags (e.g. R)

#===========================================================================

# >>> Test for senseless Contact Distance s.u.  in CIF

_754  0.0 99.0 99.0 4 A

Contact Calc$X, Rep$Y ......  Senseless su

An s.u. should not be given in the CIF for constrained contact distances.

Please check for proper refinement status flags (e.g. R)

#===========================================================================

# >>> Test for senseless  H-Bond D-H distance s.u. in CIF

_755  0.0 99.0 99.0 4 A

D-H     Calc$X, Rep$Y ......  Senseless su

An s.u. should not be given in the CIF for constrained distances.

Please check for proper refinement status flags (e.g. R)

#===========================================================================

# >>> Test for senseless  H-Bond H..A Distance s.u. in CIF

_756  0.0 99.0 99.0 4 A

H...A   Calc$X, Rep$Y ......  Senseless su

An s.u. should not be given in the CIF for constrained distances.

Please check for proper refinement status flags (e.g. R)

#===========================================================================

# >>> Test for senseless  H-Bond D..A Distance s.u. in CIF

_757  0.0 99.0 99.0 4 A

D...A   Calc$X, Rep$Y ......  Senseless su

An s.u. should not be given in the CIF for constrained distances.

Please check for proper refinement status flags (e.g. R)

#===========================================================================

# >>> Test for Senseless  H-Bond D-H..A Angle s.u. in CIF

_758  0.0 99.0 99.0 4 A

D-H..A  Calc$X, Rep$Y ......  Senseless su

An s.u. should not be given in the CIF for constrained angles.

Please check for proper refinement status flags (e.g. R)

#===========================================================================

# >>> Test for the presence of at least one torsion angle in the CIF

_760    0    2    2 1 I

CIF Contains no Torsion Angles .................          ?

The CIF contains no torsion angle entries. This might be accomplished with

the SHELXL instruction CONF.

Inclusion is encouraged by Acta Cryst. but not necessarily so by other

journals.

#===========================================================================

# >>> Test for the presence of at least one X-H in the CIF

_761    0    0    0 1 I

CIF Contains no X-H Bonds ......................          ?

The CIF contains no X-H Bonds. This might be caused by not using the

SHELXL instruction BOND $H

Inclusion is required by Acta Cryst. but not necessarily so by other journals.

#===========================================================================

# >>> Test for at least one X-Y-H or H-Y-H entry in the CIF

_762    0    0    0 1 I

CIF Contains no X-Y-H or H-Y-H Angles ..........          ?

The CIF contains no X-Y-H or H-Y-H bond angles. This might be caused by not

using the SHELXL instruction BOND $H. Those data should also be supplied

when H-atoms are introduced on calculated positions and/or refined riding

on their carrier atom.

Inclusion is required by Acta Cryst. but not necessarily so by other journals.

#===========================================================================

# >>> Test for missing bonds in CIF

_763 0.01  0.1  5.0 1 I

Incomplete CIF Bond list Detected (Rep/Expd) ... $F Ratio

Bond list in CIF likely incomplete.

#===========================================================================

# >>> Test for overcomplete bonds in CIF

_764  0.0  5.0  5.0 4 I

Overcomplete CIF Bond List Detected (Rep/Expd) . $F Ratio

The CIF contains more bonds than the unique set, indicating redundancy.

An example is redundancy due to the inclusion of symmetry related bonds.

#===========================================================================

# >>> Test for suspect C-H bonds in CIF (Not caught otherwise)

_770  0.0  0.0  0.0 2 A

Suspect C-H Bond in CIF:   $A--  $B .. $F Ang.

Report on unusual C-H bonds not caught in other tests.

#===========================================================================

# >>> Test for suspect N-H bonds in CIF (Not caught otherwise)

_771  0.0  0.0  0.0 2 A

Suspect N-H Bond in CIF:   $A--  $B .. $F Ang.

Report on unusual N-H bonds not caught in other tests.

#===========================================================================

# >>> Test for suspect O-H bonds in CIF (Not caught otherwise)

_772  0.0  0.0  0.0 2 A

Suspect O-H Bond in CIF:   $A--  $B .. $F Ang.

Report on unusual O-H bonds not caught in other tests.

Note: Exceptions can be H-atoms in acid O..H..O bridges or in H5O2+

(Hydronium) species.

#===========================================================================

# >>> Test for suspect C-C bonds in CIF (Not caught otherwise)

_773  0.0  0.0  0.0 2 A

Suspect C-C Bond in CIF:   $A--  $B .. $F Ang.

Report on unusual C-C bonds not caught in other tests.

#===========================================================================

# >>> Test for too large / erroneous bond distance

_774  4.0  6.0  9.0 1 A

Suspect X-Y Bond in CIF:   $A--  $B .. $F Ang.

Likely Erroneous Bond Entry

#===========================================================================

# >>> Test for too large / erroneous Contact distance

_775  4.0  6.0  9.0 1 A

Suspect X-Y Cont in CIF:   $A--  $B .. $F Ang.

Likely Erroneous Contact Entry

#===========================================================================

# >>> Test for suspect Angle in CIF (Not caught otherwise)

_779  0.0  0.0  0.0 4 I

Suspect or Irrelevant (Bond) Angle in CIF ...... $F Deg.

Possibly erroneous (Bond)angle less than 45 degree. The angle might be

considered for elimination from the CIF when irrelevant. This ALERT can

also be triggered when the assigned occupancy factors are incorrect.

#===========================================================================

# >>> Test whether coordinates form a connected set

_780    0    0    0 1 A

Coordinates do not Form a Properly Connected Set          ?

Atoms given in a CIF should form a 'connected set', i.e. no symmetry

operations are needed to get atoms in a bonding position. A connected set of

atoms is not needed for the least squares refinement (unless hydrogen atoms

are to be added at calculated positions). Geometry listings (bonds, angles,

torsions & H-bonds) become unwieldy for non-connected atom sets.

#===========================================================================

# >>> Test for Flack x value for Centrosymmetric space-group

_781    0    0    0 1 A

Flack Parameter is given for Centro Space-group.          ?

A Flack parameter value is erroneously given for a structure reported

in a centrosymmetric space-group.

#===========================================================================

# >>> Test Whether C.G. Residue in Unitcell Box

_790    0    2    2 4 I

Centre of Gravity not Within Unit Cell: Resd.  # $I

Unless for a good reason, molecular species should be transformed

(by symmetry and/or translation) so that their centres of gravity

are close to or within the unitcell bounds. This is a strict rule for the

main species. Deviations from this general rule are for smaller additional

species when relevant for intermolecular interactions with the main species.

#---------------------------------------------------------------------------

# >>> Check the absolute configuration of chiral atom in 'chiral' spgr

_791    0    0    0 4 I

The Model has Chirality at $A (Verify) ....          $B

This test addresses the consistency of the absolute configuration

assignment in non-centrosymmetric structures with proper symmetry

operations (i.e. all matrices with determinant = 1) only.

Verify the (R/S) absolute configuration assignment of this atom and

the consistency of the absolute configuration implicit in the CIF-data

with that in the 'ORTEP' illustration. Torsion angles should have the

correct sign.

The absolute structure assigment should also be consistent with the

lowest value of the Flack parameter and/or know absolute configuration.

#---------------------------------------------------------------------------

# >>> Check the absolute configuration of chiral atom in 'non-chiral' spgr

_792    0    0    0 1 I

The Model has Chirality at $A (Verify) ....          $B

This test addresses the consistency of the absolute structure assignment

(i.e. polarity etc.) in non-centrosymmetric structures in space groups

that include improper symmetry operations (e.g. mirror planes).

Check the (R/S) absolute configuration assignment of this atom and

the consistency of the absolute configuration implicit in the CIF-data

with that in the 'ORTEP' illustration.

#---------------------------------------------------------------------------

# >>> Check the absolute configuration of chiral atom in centrosymmetric spgr

_793    0    0    0 4 A

The Model has Chirality at $A (Verify) ....          $B

This test addresses the consistency of the absolute configuration assignment

of molecules in the reported asymmetric unit among coordinates,

molecular presentations and chemical diagrams.

Check the (R/S) absolute configuration assignment of this atom and

the consistency of the absolute configuration implicit in the CIF-data

with that in the 'ORTEP' illustration.

#---------------------------------------------------------------------------

# >>> Report the calculated 'Valence Bond' valency for metals

_794    0    0    0 5 A

Check Predicted Bond Valency for $A $B $F

This test reports the valency of an atom as predicted by the Valence

Bond Model. See:

N.E. Brese & M. O'Keeffe (1991) Acta Cryst. B47, 192-197.

I.D. Brown (2002). The Chemical Bond in Inorganic Chemistry:

The Bond Valence Model. Oxford University Press.

More explicit info on the calculations can be obtained by running the

calculations explicitly with the PLATON option BondValence.

Note: The underlying theory is empirical and might not apply to the

case at hand (e.g. charged species).

#===========================================================================

# >>> Test for C-atom Labels Ordered

_795  0.0  0.0  0.0 4 I

C-Atom in CIF Coordinate List out of Sequence ..    $A

Atoms in the CIF are not given in logical order (i.e. C1, C2 etc.)

The recommended procedure is to sort the atom list in some logical

order before the final refinement cycles.

#===========================================================================

# >>> Test for O-atom Labels Ordered

_796  0.0  2.0  2.0 4 I

O-Atom in CIF Coordinate List out of Sequence ..    $A

Atoms in the CIF are not given in logical order (i.e. O1, O2 etc.)

The recommended procedure is to sort the atom list in some logical

order before the final refinement cycles.

#===========================================================================

# >>> Test for N-atom Labels Ordered

_797  0.0  2.0  2.0 4 I

N-Atom in CIF Coordinate List out of Sequence ..    $A

Atoms in the CIF are not given in logical order (i.e. N1, N2 etc.)

The recommended procedure is to sort the atom list in some logical

order before the final refinement cycles.

#===========================================================================

# >>> Test for Alphanumeric Label on coordinate record

_798  0.0  2.0  2.0 4 A

Numeric Atom Label on Coordinate Par. Record ...    $A

Atom labels are generally not a number (i.e. starting with one or two

characters indicating the atom type). Labels can be erroneously numeric

due to typing errors (e.g. 'Oxygen' typed as 'zero').

#===========================================================================

# >>> Test for Alphanumeric Label on displacement par. record

_799  0.0  2.0  2.0 4 A

Numeric Label on Displacement Par. Record ......    $A

Atom labels are generally not a number (i.e. starting with one or two

characters indicating the atom type). Labels can be erroneously numeric

due to typing errors (e.g. 'Oxygen' typed as 'zero').

#===========================================================================

# >>> Test for missing, incomplete or out-of-order Cell data

_801  0.0  0.0  0.0 4 A

Cell Data Missing, Incomplete or Out-of-Order             ?

PLATON has a problem with the Cell data. The possible reason can be that

the cell data are missing, incomplete or out-of-sequence. PLATON/CHECK wishes

to see the cell and symmetry data before any coordinates are given.

PLATON expects the values of all six cell parameters.

#===========================================================================

# >>> Test for Input lines longer than 80 Characters

_802    0   10   10 4 A

CIF Input Record(s) with more than 80 Characters          !

The CIF contains records longer than 80 characters. Not all software will

read beyond column 80.

The CIF-1.1 definition specifies a maximum of 2048 character per record.

#===========================================================================

# >>> Test for Loop problem in CIF-Read

_803  0.0  0.0  0.0 1 A

Loop Problem in CIF-Reading (Too Many loop Items)         !

Fatal Problem: Check loop datanames and data for errors. There are

likely too many or to few data in the loop.

#===========================================================================

# >>> Test for ARU-Pack Problem in PLATON

_804    0    0    0 5 I

ARU-Pack Problem in PLATON Analysis ............ $I Times

Problem: ARU representations turn out to be needed outside the ORTEP

style -5:5 unitcell translation range. The Analysis might be incomplete.

The problem often occurs for structures with aliphatic chains stretching

over many unitcells or network structures. Transformation of the unitcell

content to a symmetry related position might solve the problem.

#===========================================================================

# >>> Test for insufficient 'coordinate data'

_805    0    0    0 1 A

Fatal Problem: Insufficient Data in Atom Loop ..          !

Check Coordinate Data Loop.

#===========================================================================

# >>> Test for insufficient 'UIJ data'

_806    0    2    2 4 A

Fatal Problem: Insufficient Data in UIJ Loop ...          !

Check UIJ Data Loop.

#===========================================================================

# >>> Test for Maximum number of ATOMS Exceeded Problem

_807    0    0    0 5 A

Fatal Problem: Maximum Number of Atoms Exceeded.          !

PLATON can handle up to 7000 atoms in the (expanded) ATOM list.

This might happen with disordered or network structures in high

symmetry space groups. Deletion of the symmetry information might solve

part of the problem and provide a partial validation.

#===========================================================================

# >>> Test for Out-of-Memory Problem

_810    0    0    0 5 A

Out-of-Memory Problem in PLATON/ASYM ...........          !

Analysis for missing reflections may be incomplete due to an out-of-memory

problem.

#===========================================================================

# >>> Test for No ADDSYM Analysis

_811    0    2    2 5 A

No ADDSYM Analysis: Too Many Excluded Atoms ....          !

The ADDSYM test for missed symmetry is not executed for structures with too

many disordered atoms.

#===========================================================================

# >>> Test for BASF/TWIN Problem in SHELXL

_850  0.0  2.0  2.0 4 A

Check Flack Parameter Exact Value 0.00 and su .. $F

This G_ALERT can be ignored in the case that the so-called 'on-the-cheap'

Flack parameter is reported as determined with SHELXL.

Exactly zero values are possible but may also be a software artifact.

The following should be checked:

Problem #1: Some SHELXL97 versions do not allow negative values of the

Flack parameter when determined using the BASF/TWIN instructions.

Negative values are set to 0.00001. Refinement may not converge completely.

Problem #2: Some SHELXL97 versions put meaningless values in the CIF for the

Flack parameter when 'TWIN -1 0 0 0 -1 0 0 0 -1 2 / BASF' instructions (i.e.

an explicit matrix is specified on the TWIN instruction) are used.

Please check the value of BASF (in the list output)  against the Flack

parameter in the CIF.

#=====================================================================

# >>> Test for restraints used in refinement

_860    0    0    0 3 I

Note: Number of Least-Squares Restraints ....... $I

The use of restraints used in the refinement should be explained in

the write-up of a structure analysis.

Note: An exception are restraints for floating origins (e.g. in P21).

#=====================================================================

# >>> Test for 'No-matching reflection file'

_900    0    0    0 1 I

No Matching Reflection File Found ..............          !

Likely cause: Dataset names on CIF and FCF differ.

Note: FCF Validation is Skipped for this Entry.

#=====================================================================

# >>> Test for CIF & FCF CELL Not Matching

_901    0    0    0 1 I

Cell Parameters in CIF and FCF do not Match ....          !

Likely causes: Wrong Dataset, CIF or FCF Parameters Edited inconsistently.

Note: FCF Validation is Skipped for this Entry.

#=====================================================================

# >>> Test for non-zero number of recognised reflections on FCF

_902    0    0    0 1 I

No (Interpretable) Reflections found in FCF ....          !

Either no reflections in FCF or uninterpretable due to unknown

format or editing.

Note: FCF Validation is Skipped for this Entry.

#=====================================================================

# >>> Test for missing reflections below Theta-Min

_910    0   10   50 3 A

Missing # of FCF Reflections Below Th(Min) ..... $I

Possible causes: Beamstop theta-min limit set too high, large unitcell etc.

A possible technical solution on CCD based equipment involves the collection

of additional images with the detector at a larger distance from the

crystal with the beamstop setting changed accordingly..

#=====================================================================

# >>> Test for missing reflections between Theta-Min and sinth/lambda=0.6

_911   -1    5  999 3 A

Missing # FCF Refl Between THmin & STh/L=$A $I

Possible causes: Missing cusp of data (due to rotation about one axis),

deleted (overflow) reflections or improper stategy (orthorhombic for

monoclinic crystal etc.)

#=====================================================================

# >>> Test for missing reflections above Sin(TH)/Lambda = 0.6

_912   -1  100  999 4 A

Missing # of FCF Reflections Above STh/L=  0.600 $I

Possible causes: Missing cusp of data (due to rotation about one axis),

deleted (overflow) reflections or improper stategy (orthorhombic for

monoclinic crystal etc.)

#=====================================================================

# >>> Test for missing strong reflections

_913    0  100  999 3 A

Missing # of Very Strong Reflections in FCF .... $I

This ALERT reports the number of missing reflections with Fc**2 values

greater than the largest Fc**2 value in the FCF.

Possible causes: Missing cusp of data (due to rotation about one axis),

deleted (overflow) reflections or improper stategy (orthorhombic for

monoclinic crystal etc.) or behind the beamstop.

#=====================================================================

# >>> Test for absence of Bijvoet Pairs in non-centro structure

_914    0    2    2 3 A

No Bijvoet Pairs in FCF for Non-centro Structure          !

This ALERT reflects the notion that a dataset should contain a sufficient

number of Bijvoet (Friedel) pairs for the reliable determination of the

absolute structure of a non-centrosymmetric crystal structure.

This test is invoked when a Flack parameter value is specified.

Note: SHELXL97 will calculate/report a Flack parameter value even for

refinement against Friedel merged data. Remove the Flack entry from the

CIF.

#=====================================================================

# >>> Test for low Friedel Pair Coverage in non-centro structure

_915  10.  50.  100 3 A

Low Friedel Pair Coverage ...................... $F Perc

This ALERT reflects the notion that a dataset should contain a sufficient

number of Bijvoet (Friedel) pairs for the reliable determination of the

absolute structure of a non-centrosymmetric crystal structure.

A Friedel coverage that deviates significantly from 100 procent may

bias/invalidate the value of the Flack parameter.

#=====================================================================

# >>> Test for differing Flack x and Hooft y Parameter values

_916  0.1  0.2  0.3 2 A

Hooft y and Flack x Parameter values differ by . $F

The Hooft y Parameter is calculated independently from the Bijvoet

differences and should have a value similar (observing the su's) to that of

the Flack x Parameter.

See:

Hooft, R.W.W, Straver, L.H. & Spek,A.L. (2008). J. Appl, Cryst. 41, 96-103.

Large differences may arise in cases where the Flack parameter was not

done with BASF/TWIN or with essentially centrosymmetric data.

See:

Flack, H.D., Bernardinelli, G, Clemente, D.A., Linden, A. Spek, A.L. (2006)

Acta Cryst. B62, 695-701.

#=====================================================================

# >>> Test for reflections effected by the beamstop

_919    0    0  999 3 A

Reflection(s) # Likely Affected by the Beamstop  $I

This ALERT reports the number of reflections with intensities seriously

effected by the beamstop. Reflections are counted for which theta < 3

Degrees and (Fo**2 - Fc**2) / sqrt(weight) < - 10.0. Those reflections

are better removed from the final refinement since they are in systematic

error.

#=====================================================================

# >>> Test for TH(Max) Consistency between CIF & FCF

_920  .10  .50  1.0 1 A

Theta(Max)  in CIF and FCF Differ by ........... $F Deg.

Check reflection statistics of the data in the FCF for consistency with

the data reported in the CIF.

A difference usually indicates an edited CIF or an FCF file that was

not created in the same SHELXL run where the CIF was created.

#=====================================================================

# >>> Test for R1 Consistency between CIF & FCF(Reported)

_921  .10  .50  5.0 1 A

R1  * 100.0 in the CIF and FCF Differ by ....... $F

Please check whether the supplied FCF corresponds with the CIF produced in

the same least squares refinement job.

The test is based on the observed and calculated F**2 in the FCF and de

weight parameters taken from the CIF.

#=====================================================================

# >>> Test for wR2 Consistency between CIF & FCF(Reported)

_922  .10  .50  5.0 1 A

wR2 * 100.0 in the CIF and FCF Differ by ....... $F

Please check whether the supplied FCF corresponds with the CIF produced in

the same least squares refinement job.

The test is based on the observed and calculated F**2 in the FCF and de

weight parameters taken from the CIF.

#=====================================================================

# >>> Test for S Consistency between CIF & FCF(Reported)

_923  .01  .02  .03 1 A

S    values in the CIF and FCF Differ by ....... $F

Please check whether the supplied FCF corresponds with the CIF produced in

the same least squares refinement job.

The test is based on the observed and calculated F**2 in the FCF and de

weight parameters taken from the CIF.

#=====================================================================

# >>> Test for Consistency of the  Reported & Calculated  Rho(min)

_924  1.0  2.0  3.0 1 A

The Reported and Calculated Rho(min) Differ by . $F eA-3

Check & Explain why the Reported Rho(min) differs significantly from

the value calculated on the basis of the reported structure.

Note: The Reported and Calculated values may differ slightly due to

a differing peak interpolation algorithm.

#=====================================================================

# >>> Test for  Consistency of the Reported & Calculated Rho(max)

_925  1.0  2.0  3.0 1 A

The Reported and Calculated Rho(max) Differ by . $F eA-3

Check & Explain why the Reported Rho(max) differs significantly from

the value calculated on the basis of the reported structure.

Note: The Reported and Calculated values may differ slightly due to

a differing peak interpolation algorithm.

#=====================================================================

# >>> Test for R1 Consistency between CIF & FCF(Calculated)

_926  .10  .50  5.0 1 A

Reported and Calculated  R1 * 100.0 Differ by .. $F

Please check whether the R1 value that is reported in the CIF corresponds

with the R1 value calculated from the parameters supplied in the CIF.

This test is based on the observed reflection data in the FCF and

reflection data that are calculated with the parameters (i.e. coordinates,

displacement and weight parameters) taken from the CIF.

#=====================================================================

# >>> Test for wR2 Consistency between CIF & FCF(Calculated)

_927  .10  .50  5.0 1 A

Reported and Calculated wR2 * 100.0 Differ by .. $F

Please check whether the wR2 value that is reported in the CIF corresponds

with the wR2 value calculated from the parameters supplied in the CIF.

This test is based on the observed reflection data in the FCF and

reflection data that are calculated with the parameters (i.e. coordinates,

displacement and weight parameters) taken from the CIF.

#=====================================================================

# >>> Test for S Consistency between CIF & FCF(Calculated)

_928  .10  .20 2.00 1 A

Reported and Calculated  S value   Differ by ... $F

Please check whether the S value that is reported in the CIF corresponds

with the S value calculated from the parameters supplied in the CIF.

This test is based on the observed reflection data in the FCF and

reflection data that are calculated with the parameters (i.e. coordinates,

displacement and weight parameters) taken from the CIF.

#===========================================================================

# >>> Test for interpretable weight parameters

_929    0   10   10 4 A

No Interpretable (SHELX) Weight Parameters found          ?

SHELXL weight parameters are expected to be given in the format below:

_refine_ls_weighting_details

'calc w=1/[\s^2^(Fo^2^)+(0.1000P)^2^+0.0000P] where P=(Fo^2^+2Fc^2^)/3'

Do not edit this string or make it into a text block between ';'.

#=====================================================================

# >>> Test for Missed Twinning from FCF data

_930  .00  .00  .80 2 A

Check Twin Law ($A)[$B] Estimated BASF $F

Check the proposed Twin Law. The entry in () represents the proposed

rotation axis in reciprocal space and the one in [] the corresponding

rotation is direct space.

The relevant Twin Matrix can be found in the file '.fck'.

Note: This analysis is based on Fo/Fc differences with Fc data given

in the .fcf file (i.e. Fobs, Fcalc listing).

ALERT-930 is expected to generate an related ALERT-931 as well.

#=====================================================================

# >>> Test for Missed Twinning from FCF/CIF data

_931  .00  .00  .80 5 A

Check Twin Law ($A)[$B] Estimated BASF $F

Check the proposed Twin Law. The entry in () represents the proposed

rotation axis in reciprocal space and the one in [] the corresponding

rotation is direct space.

The relevant Twin Matrix can be found in the file '.fck'.

Note: This test is based on F(calc) values calculated with the data

in the CIF. This ALERT can be ignored when twinning has been addressed

in the refinement (As incated by the Absence of ALERT 930).

Please check whether twinning is mentioned in the write-up of the paper.

#=====================================================================

# >>> Test for wR2 refinement with all data

_940    0    0    0 3 A

F**2 Refinement with I .GT. n * Sigma(I) only ..          ?

Apparently, observed data with I > n * sigma(I) were used in the F**2

least squares refinement, rather than all observed data.

#=====================================================================

# >>> Test for Reported and Calculated Hmax Difference

_950    1  100  200 1 I

Reported and Calculated Hmax Values Differ by .. $I

Reported and Calculated Max(Hmax,-Hmin) values differ by more than one

unit. Check the consistency of wavelength and reported resolution data.

#=====================================================================

# >>> Test for Reported and Calculated Kmax Difference

_951    1  100  200 1 I

Reported and Calculated Kmax Values Differ by .. $I

Reported and Calculated Max(Kmax,-Kmin) values differ by more than one

unit. Check the consistency of wavelength and reported resolution data.

#=====================================================================

# >>> Test for Reported and Calculated Lmax Difference

_952    1  100  200 1 I

Reported and Calculated Lmax Values Differ by .. $I

Reported and Calculated Max(Lmax,-Lmin) values differ by more than one

unit. Check the consistency of wavelength and reported resolution data.

#=====================================================================

# >>> Test for Reported and Actual Hmax Difference in the FCF File

_953    1  999  999 1 I

Reported and Actual Hmax Values in FCF Differ by $I

Reported (in the CIF) and Actual (in the FCF)  Max(Hmax,-Hmin) values

differ by more than one unit. Check for data set truncation.

#=====================================================================

# >>> Test for Reported and Actual Kmax Difference in the FCF File

_954    1  999  999 1 I

Reported and Actual Kmax Values in FCF Differ by $I

Reported (in the CIF) and Actual (in the FCF) Max(Kmax,-Kmin) values

differ by more than one unit. Check for data set truncation.

#=====================================================================

# >>> Test for Reported and Actual Lmax Difference in the FCF file

_955    1  999  999 1 I

Reported and Actual Lmax Values in FCF Differ by $I

Reported (in the CIF) and Actual (in the FCF) Max(Lmax,-Lmin) values

differ by more than one unit. Check for data set truncation.

#=====================================================================

# >>> Test for reflections with I < 2 sigma

_960    0    0  999 3 A

Number of Intensities with I .LT. - 2*sig(I) ..  $I

Multiple strongly negative intensities may be indicative for poor

integration om diffraction images.

Too many negative intensities may result in higher than usual wR2 values.

#=====================================================================

# >>> Test maximum calculated residual density

_971 1.50  2.5  3.5 2 A

Large Calcd. Non-Metal Positive Residual Density $F eA-3

Larger than expected residual density maximum outside heavy atom locations.

This might be caused by unaccounted for twinning, wrongly assigned

atom types and other model errors.

#---------------------------------------------------------------------------

# >>> Test minimum calculated residual density

_972 1.50  2.5  3.5 2 A

Large Calcd. Non-Metal Negative Residual Density $F eA-3

Larger than expected residual density minimum outside heavy atom locations.

This might be caused by unaccounted for twinnings, wrongly assigned

atom types and other model errors.

#===========================================================================

$ END-OF-SECTION